Chapter 2 – Data Representation

The focus of this chapter is the representation of data in a digital computer.  We begin with a 
review of several number systems (decimal, binary, octal, and hexadecimal) and a discussion 
of methods for conversion between the systems.  The two most important methods are 
conversion from decimal to binary and binary to decimal.  The conversions between binary 
and each of octal and hexadecimal are quite simple.  Other conversions, such as hexadecimal 
to decimal, are often best done via binary.

After discussion of conversion between bases, we discuss the methods used to store integers 
in a digital computer: one’s complement and two’s complement arithmetic.  This includes a 
characterization of the range of integers that can be stored given the number of bits allocated 
to store an integer.  The most common integer storage formats are 16 and 32 bits.

The next topic for this chapter is the storage of real (floating point) numbers.  This discussion 
will focus on the standard put forward by the Institute of Electrical and Electronic Engineers, 
the IEEE Standard 754 for floating point numbers.  The chapter closes with a discussion of 
codes for storing characters: ASCII, EBCDIC, and Unicode.

Number Systems

There are four number systems of possible interest to the computer programmer: decimal, 
binary, octal, and hexadecimal.  Each system is characterized by its base or radix, always 
given in decimal, and the set of permissible digits.  Note that the hexadecimal numbering 
system calls for more than ten digits, so we use the first six letters of the alphabet.

Decimal	Base = 10

Digit Set = {0, 1, 2, 3, 4, 5, 6, 7, 8, 9}

Binary	Base = 2

Digit Set = {0, 1}

Octal	Base = 8 = 23

Digit Set = {0, 1, 2, 3, 4, 5, 6, 7}

Hexadecimal	Base = 16 = 24

Digit Set = {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, A, B, C, D, E, F}

The fact that the bases for octal and hexadecimal are powers of the basis for binary facilitates 
the conversion between these bases.  The conversion can be done one digit at a time, 
remembering that each octal digit corresponds to three binary bits and each hexadecimal digit 
corresponds to four binary bits.  Conversion between octal and hexadecimal is best done by 
first converting to binary.

Except for an occasional reference, we shall not use the octal system much, but focus on the 
decimal, binary, and hexadecimal numbering systems.

The figure below shows the numeric equivalents in binary, octal, and decimal of the first 16 
hexadecimal numbers.  If octal numbers were included, they would run from 00 through 017.

Binary	Decimal	Hexadecimal
(base 2)	(base 10)	(base 16)

0000	00	0

0001	01	1	Note that conversions from hexadecimal

0010	02	2	to binary can be done one digit at a time,

0011	03	3	thus DE = 11011110, as D = 1101 and

0100	04	4	E = 1110.  We shall normally denote

0101	05	5	this as DE = 1101 1110 with a space

0110	06	6	to facilitate reading the binary.

0111	07	7

1000	08	8	Conversion from binary to hexadecimal

1001	09	9	is also quite easy.  Group the bits four at

1010	10	A	a time and convert each set of four.

1011	11	B	Thus 10111101, written 1011 1101 for

1100	12	C	clarity is BD because 1011 = B and

1101	13	D	1101 = D.

1110	14	E

1111	15	F

Consider conversion of the binary number 111010 to hexadecimal.  If we try to group the bits 
four at a time we get either 11 1010 or 1110 10.  The first option is correct as the grouping 
must be done from the right.  We then add leading zeroes to get groups of four binary bits, 
thus obtaining 0011 1010, which is converted to 3A as 0011 = 3 and 1010 = A.

Unsigned Binary Integers

There are two common methods to store unsigned integers in a computer: binary numbers 
(which we discuss now) and Packed Decimal (which we discuss later).  From a theoretical 
point of view, it is important to note that no computer really stores the set of integers in that it 
can represent an arbitrary member of that infinite set.  Computer storage formats allow only 
for the representation of a large, but finite, subset of the integers.

It is easy to show that an N–bit binary integer can represent one of 2N possible integer values.  
Here is the proof by induction.

1.	A one–bit integer can store 2 values: 0 or 1.  This is the base for induction.

2.	Suppose an N–bit integer, unconventionally written as BNBN–1 … B3B2B1.
		By the inductive hypothesis, this can represent one of 2N possible values.

3.	We now consider an (N+1)–bit integer, written as BN+1BNBN–1 … B3B2B1.
		By the inductive hypothesis, there are 2N values of the form 0BNBN–1 … B3B2B1,
		and 2N values of the form 1BNBN–1 … B3B2B1.

4.	The total number of (N+1)–bit values is 2N + 2N = 2N+1.  The claim is proved.

By inspection of the above table, we see that there are 16 possible values for a four–bit 
unsigned integer.  These range from decimal 0 through decimal 15 and are easily represented 
by a single hexadecimal digit.  Each hexadecimal digit is shorthand for four binary bits.

In the standard interpretation, always used in this course, an N–bit unsigned integer will 
represent 2N integer values in the range 0 through 2N – 1, inclusive.  Sample ranges include:

N =	4	0 through	24 – 1	0 through	15
	N =	8	0 through	28 – 1	0 through	255
	N =	12	0 through	212 – 1	0 through	4095
	N =	16	0 through	216 – 1	0 through	65535
	N =	20	0 through	220 – 1	0 through	1,048,575
	N =	32	0 through	232 – 1	0 through	4,294,967,295

For most applications, the most important representations are 8 bit, 16 bit, and 32 bit.  To this 
mix, we add 12–bit unsigned integers as they are used in the base register and offset scheme 
of addressing used by the IBM Mainframe computers.  Recalling that a hexadecimal digit is 
best seen as a convenient way to write four binary bits, we have the following.

8 bit numbers	2 hexadecimal digits	0 through	255,
	12 bit numbers	3 hexadecimal digits	0 through	4095,
	16 bit numbers	4 hexadecimal digits	0 through	65535,	and
	32 bit numbers	8 hexadecimal digits	0 through	4,294,967,295.

Conversions between Decimal and Binary

We now consider methods for conversion from decimal to binary and binary to decimal. We 
consider not only whole numbers (integers), but numbers with decimal fractions.  To convert 
such a number, one must convert the integer and fractional parts separately.

Consider the conversion of the number 23.375.  The method used to convert the integer part 
(23) is different from the method used to convert the fractional part (.375).  We shall discuss 
two distinct methods for conversion of each part and leave the student to choose his/her 
favorite.  After this discussion we note some puzzling facts about exact representation of 
decimal fractions in binary; e.g. the fact that 0.20 in decimal cannot be exactly represented in 
binary.  As before we present two proofs and let the student choose his/her favorite and 
ignore the other.

The intuitive way to convert decimal 23 to binary is to note that 23 = 16 + 7 = 16 + 4 + 2 + 1; 
thus decimal 23 = 10111 binary.  As an eight bit binary number, this is 0001 0111.  Note that 
we needed 5 bits to represent the number; this reflects the fact that 24 < 23  25.  We expand 
this to an 8-bit representation by adding three leading zeroes.

The intuitive way to convert decimal 0.375 to binary is to note that 0.375 = 1/4 + 1/8 =
0/2 + 1/4 + 1/8, so decimal .375 = binary .011 and decimal 23.375 = binary 10111.011.

Most students prefer a more mechanical way to do the conversions.  Here we present that 
method and encourage the students to learn this method in preference to the previous.

Conversion of integers from decimal to binary is done by repeated integer division with 
keeping of the integer quotient and noting the integer remainder.  The remainder numbers are 
then read top to bottom as least significant bit to most significant bit.  Here is an example.

Quotient	Remainder

23/2 = 	11	1	Thus decimal 23 = binary 10111

11/2 = 	5	1

5/2 =	2	1	Remember to read the binary number

2/2 = 	1	0	from bottom to top.

1/2 = 	0	1

Conversion of the fractional part is done by repeated multiplication with copying of the 
whole number part of the product and subsequent multiplication of the fractional part.  All 
multiplications are by 2.  Here is an example.

Number	Product	Binary

0.375	x 2 =	0.75	0

0.75	x 2 =	1.5	1

0.5	x 2 =	1.0	1

The process terminates when the product of the last multiplication is 1.0.  At this point we 
copy the last 1 generated and have the result; thus decimal 0.375 = 0.011 binary.

We now develop a “power of 2” notation that will be required when we study the IEEE 
floating point standard.  We have just shown that decimal 23.375 = 10111.011 binary.  Recall 
that in the scientific “power of 10” notation, when we move the decimal to the left one place 
we have to multiply by 10.  Thus, 1234 = 123.4  101 = 12.34  102 = 1.234  103.

We apply the same logic to the binary number.  In the IEEE standard we need to form the 
number as a normalized number, which is of the form 1.xxx  2p.  In changing 10111 to
1.0111 we have moved the decimal point (O.K. – it should be called binary point) 4 places to 
the left, so 10111.011 = 1.0111011  24.  Recalling that 24 = 16 and 25 = 32, and noting that 
16.0 < 23.375  32.0 we see that the result is as expected.

Conversion from binary to decimal is quite easy.  One just remembers the decimal 
representations of the powers of 2.  We convert 10111.011 binary to decimal.  Recalling the 
positional notation used in all number systems:

10111.011	= 124 + 023 + 122 + 121 + 120 + 02-1 + 12-2 + 12-3

= 116 + 08 + 14 + 12 + 11 + 00.5 + 10.25 + 10.125
			= 23.375

# Conversions between Decimal and Hexadecimal

The conversion is best done by first converting to binary.  We consider conversion of 23.375 
from decimal to hexadecimal.  We have noted that the value is 10111.011 in binary.

To convert this binary number to hexadecimal we must group the binary bits in groups of 
four, adding leading and trailing zeroes as necessary.  We introduce spaces in the numbers in 
order to show what is being done.

10111.011 = 1 0111.011.

To the left of the decimal we group from the right and to the right of the decimal we group 
from the left.  Thus 1.011101 would be grouped as 1.0111 01.

At this point we must add extra zeroes to form four bit groups.  So

10111.011 = 0001 0111.0110.

Conversion to hexadecimal is done four bits at a time.  The answer is 17.6 hexadecimal.

Another Way to Convert Decimal to Hexadecimal

Some readers may ask why we avoid the repeated division and multiplication methods in 
conversion from decimal to hexadecimal.  Just to show it can be done, here is an example.

Consider the number 7085.791748046875.  As an example, we convert this to hexadecimal.

The first step is to use repeated division to produce the whole–number part.

7085 / 16	= 442	with remainder = 13	or hexadecimal D

442 / 16	= 27	with remainder = 10	or hexadecimal A
	27 / 16	= 1	with remainder = 11	or hexadecimal B
	1 / 16	= 0	with remainder = 1	or hexadecimal 1.

The whole number is read bottom to top as 1BAD.

Now we use repeated multiplication to obtain the fractional part.

0.791748046875  16 = 	12.6679875	Remove the 12	or hexadecimal C

0.6679875  16 = 	10.6875	Remove the 10	or hexadecimal A

0.6875  16 = 	11.00	Remove the 11	or hexadecimal B

0.00  16 = 	0.0

The fractional part is read top to bottom as CAB.  The hexadecimal value is 1BAD.CAB, 
which is a small joke on the author’s part.  The only problem is to remember to write 
results in the decimal range 10 through 15 as hexadecimal A through F.

Long division is of very little use in converting the whole number part.  It does correctly 
produce the first quotient and remainder.  The intermediate numbers may be confusing.

442

16)7085

64

68

64

45

32

13

# Non-terminating Fractions

We now make a detour to note a surprising fact about binary numbers – that some fractions 
that terminate in decimal are non-terminating in binary.  We first consider terminating and 
non-terminating fractions in decimal.  All of us know that 1/4 = 0.25, which is a terminating 
fraction, but that 1/3 = 0.333333333333333333333333333333…, a non-terminating fraction.

We offer a demonstration of why 1/4 terminates in decimal notation and 1/3 does not, and 
then we show two proofs that 1/3 cannot be a terminating fraction.

Consider the following sequence of multiplications
	¼  10 = 2½
	½  10 = 5.  Thus 1/4 = 25/100 = 0.25.

Put another way, ¼ = (1/10)  (2 + ½) =  (1/10)  (2 + (1/10)  5).

However, 1/3  10 = 10/3 = 3 + 1/3, so repeated multiplication by 10 continues to yield a 
fraction of 1/3 in the product; hence, the decimal representation of 1/3 is non-terminating.

Explicitly, we see that 1/3 = (1/10)  (3 + 1/3) = (1/10)  (3 + (1/10)  (3 + 1/3)), etc.

In decimal numbering, a fraction is terminating if and only if it can be represented in the 
form J / 10K for some integers J and K.  We have seen that 1/4 = 25/100 = 25/102, thus the 
fraction 1/4 is a terminating fraction because we have shown the integers J = 25 and K = 2.

Here are two proofs that the fraction 1/3 cannot be represented as a terminating fraction in 
decimal notation.  The first proof relies on the fact that every positive power of 10 can be 
written as 9M + 1 for some integer M.  The second relies on the fact that 10 = 25, so that 
10K = 2K5K.  To motivate the first proof, note that 100 = 1 = 90 + 1, 10 = 91 + 1,

100 = 911 + 1, 1000 = 9111 + 1, etc.  If 1/3 were a terminating decimal, we could solve the 
following equations for integers J and M.

13=J10K=J9⋅M+1, which becomes 3J = 9M + 1 or 3(J – 3M) = 1.  But there is no 
integer X such that 3X = 1 and the equation has no integer solutions.

The other proof also involves solving an equation.  If 1/3 were a non-terminating fraction, 
then we could solve the following equation for J and K.

13=J10K=J2K⋅5K, which becomes 3J = 2K5K.  This has an integer solution J only if 
the right hand side of the equation can be factored by 3.  But neither 2K nor 5K can be 
factored by 3, so the right hand side cannot be factored by 3 and hence the equation is not 
solvable.

Now consider the innocent looking decimal 0.20.  We show that this does not have a 
terminating form in binary.  We first demonstrate this by trying to apply the multiplication 
method to obtain the binary representation.

Number	Product	Binary

0.20  2 =	0.40	0
	0.40  2 =	0.80	0

0.80  2 =	1.60	1

0.60  2 =	1.20	1

0.20  2 =	0.40	0

0.40  2 =	0.80	0

0.80  2 =	1.60	1	but we have seen this – see four lines above.

So decimal 0.20 in binary is 0.00110011001100110011 …, ad infinitum.  This might 
be written conventionally as 0.00110 0110 0110 0110 0110, to emphasize the pattern.

The proof that no terminating representation exists depends on the fact that any terminating 
fraction in binary can be represented in the form J2K for some integers J and K.  Thus we 
solve 15=J2K or 5J = 2K.  This equation has a solution only if the right hand side is divisible 
by 5.  But 2 and 5 are relatively prime numbers, so 5 does not divide any power of 2 and the 
equation has no integer solution.  Hence 0.20 in decimal has no terminating form in binary.

Binary Addition

The next topic is storage of integers in a computer.  We shall be concerned with storage of 
both positive and negative integers.  Two’s complement arithmetic is the most common 
method of storing signed integers.  Calculation of the two’s complement of a number 
involves binary addition.  For that reason, we first discuss binary addition.

To motivate our discussion of binary addition, let us first look at decimal addition.  Consider 
the sum 15 + 17 = 32.  First, note that 5 + 7 = 12.  In order to speak of binary addition, we 
must revert to a more basic way to describe 5 + 7; we say that the sum is 2 with a carry-out of 
1.  Consider the sum 1 + 1, which is known to be 2.  However, the correct answer to our 
simple problem is 32, not 22, because in computing the sum 1 + 1 we must consider the 
carry-in digit, here a 1.  With that in mind, we show two addition tables – for a half-adder 
and a full-adder.  The half-adder table is simpler as it does not involve a carry-in.  
The following table considers the sum and carry from A + B.

Half-Adder A + B

A	B	Sum	Carry

0	0	0	0	Note the last row where we claim that 1 + 1 yields a

0	1	1	0	sum of zero and a carry of 1.  This is similar to the

1	0	1	0	statement in decimal arithmetic that 5 + 5 yields a

1	1	0	1	sum of 0 and carry of 1 when 5 + 5 = 10.

Remember that when the sum of two numbers equals or exceeds the value of the base of the 
numbering system (here 2) that we decrease the sum by the value of the base and generate a 
carry.  Here the base of the number system is 2 (decimal), which is 1 + 1, and the sum is 0.
Say “One plus one equals two plus zero: 1 + 1 = 10”.

For us the half-adder is only a step in the understanding of a full-adder, which implements 
binary addition when a carry-in is allowed.  We now view the table for the sum A + B, with a 
carry-in denoted by C.  One can consider this A + B + C, if that helps.

Full-Adder: A + B with Carry

A	B	C	Sum	Carry

0	0	0	0	0

0	0	1	1	0

0	1	0	1	0

0	1	1	0	1

1	0	0	1	0

1	0	1	0	1

1	1	0	0	1

1	1	1	1	1

In the next chapter, we shall investigate the construction of a full adder from digital 
gates used to implement Boolean logic.  Just to anticipate the answer, we note that the 
sum and carry table above is in the form of a Boolean truth table, which can be 
immediately converted to a Boolean expression that can be implemented in digital logic.

As an example, we shall consider a number of examples of addition of four-bit binary 
numbers.  The problem will first be stated in decimal, then converted to binary, and then 
done.  The last problem is introduced for the express purpose of pointing out an error.

We shall see in a minute that four-bit binary numbers can represent decimal numbers in the 
range 0 to 15 inclusive.  Here are the problems, first in decimal and then in binary.

1)	6 + 1	0110 + 0001

2)	11 + 1	1011 + 0001

3)	13 + 5	1101 + 0101

0110     1011     1101	In the first sum, we add 1 to an even number.  This

0001     0001     0101	is quite easy to do.  Just change the last 0 to a 1.

0111     1100     0010	Otherwise, we may need to watch the carry bits.

In the second sum, let us proceed from right to left.  1 + 1 = 0 with carry = 1.  The second 
column has 1 + 0 with carry-in of 1 = 0 with carry-out = 1.  The third column has 0 + 0 with 
a carry-in of 1 = 1 with carry-out = 0.  The fourth column is 1 + 0 = 1.

Analysis of the third sum shows that it is correct bit-wise but seems to be indicating that 
13 + 5 = 2.  This is an example of “busted arithmetic”, more properly called overflow.  
A give number of bits can represent integers only in a given range; here 13 + 5 is outside 
the range 0 to 15 inclusive that is proper for four-bit numbers.

Signed and Unsigned Integers

Fixed point numbers include real numbers with a fixed number of decimals, such as those 
commonly used to denote money amounts in the United States.  We shall focus only on 
integers and relegate the study of real numbers to the floating point discussion.

Integers are stored in a number of formats.  The most common formats today include 16 and 
32 bits.  The new edition of Visual Basic will include a 64–bit standard integer format.  
Although 32-bit integers are probably the most common, our examples focus on eight-bit 
integers because they are easy to illustrate.  In these discussions, the student should recall the 
powers of 2: 20 = 1, 21 = 2, 22 = 4, 23 = 8, 24 = 16, 25 = 32, 26 = 64, 27 = 128, and 28 = 256.

Bits in the storage of an integer are numbered right to left, with bit 0 being the right-most or 
least-significant.  In eight bit integers, the bits from left to right are numbered 7 to 0.  In 32 
bit integers, the bits from left to right are numbered 31 to 0.  Note that this is not the notation 
used by IBM for its mainframe and enterprise computers. In the IBM notation, the most 
significant bit (often the sign bit) is bit 0 and the least significant bit has the highest number; 
bit 7 for an 8–bit integer.  Here are the bit numberings for a signed 8–bit integer.

Common Notation (8–bit entry)	IBM Mainframe Notation

| Bit # | 7 | 6 | 5 – 1 | 0 |  | Bit # | 0 | 1 | 2 – 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sign | MSB |  | LSB |  |  | Sign | MSB |  | LSB |

The simplest topic to cover is the storage of unsigned integers.  As there are 2N possible 
combinations of N binary bits, there are 2N unsigned integers ranging from 0 to 2N – 1.  For 
eight-bit unsigned integers, this range is 0 though 255, as 28 = 256.  Conversion from binary 
to decimal is easy and follows the discussion earlier in this chapter.

Of the various methods for storing signed integers, we shall discuss only three
	Two’s complement
	One’s complement (but only as a way to compute the two’s complement)
	Excess 127 (for 8-bit numbers only) as a way to understand the floating point standard.

One’s complement arithmetic is mostly obsolete and interests us only as a stepping-stone to 
two’s complement arithmetic.  To compute the one’s complement of a number:

1) Represent the number as an N-bit binary number
	2) Convert every 0 to a 1 and every 1 to a 0.

Decimal 100 = 	0110 0100

One’s complement	1001 1011; in one’s complement, decimal –100 = 1001 1011 binary.

There are a number of problems in one’s complement arithmetic, the most noticeable being 
illustrated by the fact that the one’s complement of 0 is 1111 1111.  In this system, we have 
–0  0, which is a violation of some of the basic principles of mathematics.

The Two’s Complement
The two’s complement of a number is obtained as follows:

1) First take the one’s complement of the number

2) Add 1 to the one’s complement and discard the carry out of the left-most column.

Decimal 100 = 	0110 0100

One’s complement	1001 1011

We now do the addition	1001 1011
				        1
				1001 1100

Thus, in eight-bit two’s complement arithmetic

Decimal 100	= 0110 0100 binary
	Decimal – 100	= 1001 1100 binary

This illustrates one pleasing feature of two’s complement arithmetic: for both positive and 
negative integers the last bit is zero if and only if the number is even.  Note that it is essential 
to state how many bits are to be used.  Consider the 8-bit two’s complement of 100.  Now 
100 = 64 + 32 + 4, so decimal 100 = 0110 0100 binary, and we get the result above.

Consider decimal 12 = 0000 1100 binary.  If we took the two’s complement of 1100, we 
might get 0100, giving us no idea how to pad out the high–order four bits.

The real reason for the popularity of two’s complement can be seen by calculating the 
representation of – 0.  To do this we take the two’s complement of 0.

In eight bits, 0 is represented as	0000 0000

Its one’s complement is represented as	1111 1111.

We now take the two’s complement of 0.

Here is the addition	  1111 1111
			          1
			1 0000 0000 – but discard the leading 1.

Thus the two’s complement of 0 is represented as 0000 0000, as required by algebra, 
and we avoid the messy problem of having – 0  0.

In N- bit two’s complement arithmetic,	+ 127	0111 1111
the range of integers that can be 	+ 10	0000 1010
represented is – 2N-1 through 2N-1 – 1 	+1		0000 0001
inclusive, thus the range for eight-bit	0		0000 0000
two’s complement integers is –128 	– 1		1111 1111	The number is
through 127 inclusive, as 27 = 128.  The	– 10	1111 0110	negative if and
table at the right shows a number of 	– 127	1000 0001	only if the high
binary representations for this example.	– 128	1000 0000	order bit is 1.

We now give the ranges allowed for the most common two’s complement representations.
	Eight bit	– 128	to	+127
	16-bit	– 32,768	to 	+32,767
	32-bit	– 2,147,483,648	to	+2,147,483,647

The range for 64-bit two’s complement integers is – 263 to 263 – 1.  As an exercise in math, 
I propose to do a rough calculation of 263.  This will be done using only logarithms.

There is a small collection of numbers that the serious student of computer science should 
memorize.  Two of these numbers are the base-10 logarithms of 2 and 3.  To five decimal 
places, log 2 = 0.30103 and log 3 = 0.47712.

Now 263 = (100.30103)63 = 1018.9649 = 100.9649  1018 = 9.224  1018, so a 64-bit integer allows 
the representation of 18 digit numbers and most 19 digit numbers.  The precise range for 
64–bit signed integers is –9,223,372,036,854,775,808 through 9,223,372,036,854,775,807.

Reminder: For any number of bits, in two’s complement arithmetic the number is negative 
if and only if the high-order bit in the binary representation is a 1.

Sign Extension

This applies to numbers represented in one’s-complement and two’s-complement form.  The 
issue arises when we store a number in a form with more bits; for example when we store a 
16-bit integer in a 32-bit register.  The question is how to set the high-order bits.

Consider a 16-bit integer stored in two’s-complement form.  Bit 15 is the sign bit.  We can 
consider bit representation of the number as A15A14A13A12A11A10A9A8A7A6A5A4A3A2A1A0.  
Consider placing this number into a 32-bit register with bits numbered R31 through R0, with 
R31 being the sign bit.  Part of the solution is obvious: make Rk = Ak for 0  k  15.  What is 
not obvious is how to set bits 31 through 16 in R as the 16-bit integer A has no such bits.

For non-negative numbers the solution is obvious and simple, set the extra bits to 0.  This is 
like writing the number two hundred (200) as a five digit integer; write it 00200.  But 
consider the 16-bit binary number 1111 1111 1000 0101, which evaluates to decimal –123.  
If we expanded this to 0000 0000 0000 0000 1111 1111 1000 0101 by setting the high order 
bits to 0’s, we would have a positive number, evaluating as 65413.  This is not correct.

The answer to the problem is sign extension, which means filling the higher order bits of the 
bigger representation with the sign bit from the more restricted representation.  In our 
example, we set bits 31 through 16 of the register to the sign bit of the 16-bit integer.  The 
correct answer is then 1111 1111 1111 1111 1111 1111 1000 0101.

Note – the way I got the value 1111 1111 1000 0101 for the 16-bit representation of – 123 
was to compute the 8-bit representation, which is 1000 0101.  The sign bit in this 
representation is 1, so I extended the number to 16-bits by setting the high order bits to 1.

Nomenclature: “Two’s-Complement Representation” vs. “Taking the Two’s-Complement”

We now address an issue that seems to cause confusion to some students.  There is a 
difference between the idea of a complement system and the process of taking the 
complement.  Because we are interested only in the two’s-complement system, I restrict my 
discussion to that system.

Question: What is the representation of the positive number 123 in 8-bit two’s complement 
arithmetic?

Answer: 0111 1011.  Note that I did not take the two’s complement of anything to get this.

Two’s-complement arithmetic is a system of representing integers in which the two’s-
complement is used to compute the negative of an integer.  For positive integers, the method 
of conversion to binary differs from unsigned integers only in the representable range.

For N-bit unsigned integers, the range of integers representable is 0 ... 2N – 1, inclusive.  For 
N-bit two’s-complement integers the range of non-negative integers representable is

0 ... 2N-1 – 1, inclusive.  The rules for converting decimal to binary integers are the same for 
non-negative integers – one only has to watch the range restrictions.

The only time when one must use the fact that the number system is two’s-complement (that 
is – take the two’s-complement) is when one is asked about a negative number.  Strictly 
speaking, it is not necessary to take the two’s-complement of anything in order to represent a 
negative number in binary, it is only that most students find this the easiest way.

Question: What is the representation of –123 in 8-bit two’s-complement arithmetic?

Answer: Perhaps I know that the answer is 1000 0101.  As a matter of fact, I can calculate 
this result directly without taking the two’s-complement of anything, but most students find 
the mechanical way the easiest way to the solution.  Thus, the preferred solution for most 
students is

1)	We note that 0  123  27 – 1, so both the number and its negative
		can be represented as an 8-bit two’s-complement integer.

2)	We note that the representation of +123 in 8-bit binary is 0111 1011

3)	We take the two’s-complement of this binary result to get the binary
		representation of –123 as 1000 0101.

We note in passing a decidedly weird way to calculate the representations of non-negative 
integers in two’s-complement form.  Suppose we want the two’s-complement representation 
of +123 as an eight-bit binary number.  We could start with the fact that the representation of 
–123 in 8-bit two’s-complement is 1000 0101 and take the two’s complement of 1000 0101 
to obtain the binary representation of 123 = –(–123).  This is perfectly valid, but decidedly 
strange.  One could work this way, but why bother?

Summary:	Speaking of the two’s-complement does not mean that one must take the
		two’s-complement of anything.

Why Does the Two’s Complement Work?

We now ask an obvious question.  The process of taking the two’s–complement of the binary 
representation of an integer has been described and is well defined.  But why does this 
process yield the negative of the integer.  We shall work this by relying on the fact that, 
if B = –A, then A + B = 0.

We begin with a few issues of notation.  Beginning with the next chapter of the textbook, 
the plus sign, “+”, will be used mostly to denote the logical OR operation.  For the purposes 
of this discussion, it will denote addition.  In this discussion, the symbol  will denote the 
one’s–complement of the single–bit number a.  In later chapters, this will be used to refer 
to the logical NOT of the Boolean value a.  In fact, the two uses are identical; one generates 
the one’s–complement of a single bit by passing it through a NOT gate.

We now consider the bitwise addition of a binary number and its one’s–complement.  At the 
bit level, the addition table is simple, and can be represented by a half–adder.

A	 	Sum	Carry
	0	1	1	0
	1	0	1	0

Note that there are only two rows in this table, because the value of a binary bit determines 
the value of its one’s–complement.

We next prove a mathematical theorem to be used in our demonstration that the 
two’s–complement of an integer is, in fact, its negative.

Theorem: For N ≥ 0, we have

Proof:  We display this by simple mathematical induction.
We give two base cases:  1 = 20 = 21 – 1, and 1 + 2 = 20 + 21 = 3 = 22 – 1.

Now if 1 + 21 + … + 2N = 2N+1 – 1, then 
1 + 21 + … + 2N + 2N+1 = (2N+1 – 1) + 2N+1 = 22N+1 – 1 = 2N+2 – 1.

Positional Notation 
We now review the idea of positional notation for binary representations of integers, 
beginning with N–bit unsigned integers.  While the discussions are valid for all N > 0, 
we shall choose to illustrate with N = 8.

Let A be the N–bit binary number represented as AN–1AN–2 … A2A1A0.

The unsigned decimal value represented by this string is given by

.

For example, consider the 8–bit unsigned integer A = 0110 0100.
A7 = 0, A6 = 1, A5 = 1, A4 = 0, A3 = 0, A2 = 1, A1 = 0, and A0 = 0; 
the value is 027 + 126 + 125 + 024 + 023 + 122 + 021 + 020 = 
26 + 25 + 22 = 64 + 32 + 4 = 100.

We now consider N–bit patterns, interpreted as two’s–complement integers.  In this form, 
the bit pattern AN–1AN–2 … A2A1A0 represents the value given by the sum

Again, consider the 8–bit unsigned integer A = 0110 0100.
A7 = 0, A6 = 1, A5 = 1, A4 = 0, A3 = 0, A2 = 1, A1 = 0, and A0 = 0; 
the value is – 027 + 126 + 125 + 024 + 023 + 122 + 021 + 020 = 
26 + 25 + 22 = 64 + 32 + 4 = 100.

For example, consider the 8–bit unsigned integer A = 1001 1100.
A7 = 1, A6 = 0, A5 = 0, A4 = 1, A3 = 1, A2 = 1, A1 = 0, and A0 = 0; 
the value is –127 + 026 + 025 + 124 + 123 + 122 + 021 + 020 = 
–127 + 124 + 123 + 122 = –128 + 16 + 8 + 4 = –100.

Here is a proof that the two’s–complement represents the negative of a number.  We 
shall have a bit more to say about the proof after we give it.  Let A be represented by:

If AN–1=1, the number is negative.  If AN–1=0, the number is not negative.

Let B = – A, as represented in the two’s–complement notation.  B represents the value:

But we have shown that for every bit index that

So we have the following.

In order to illustrate this more fully, we consider the negative range for four–bit 
integers represented in two’s–complement form.  The range is from –8 to –1 inclusive.

Here is the table with the results for discussion.

| Decimal | Positive | One’s Complement | Two’s Complement | Comment |
| --- | --- | --- | --- | --- |
| 8 |  |  | 1000 | –8 |
| 7 | 0111 | 1000 | 1001 | –8 + 1 |
| 6 | 0110 | 1001 | 1010 | –8 + 2 |
| 5 | 0101 | 1010 | 1011 | –8 + 3 |
| 4 | 0100 | 1011 | 1100 | –8 + 4 |
| 3 | 0011 | 1100 | 1101 | –8 + 5 |
| 2 | 0010 | 1101 | 1110 | –8 + 6 |
| 1 | 0001 | 1110 | 1111 | –8 + 7 |

The above presents an interesting argument and proof, but it overlooks one essential point.  
How does the hardware handle this?  For any sort of adder, the bits are just that.  There is 
nothing special about the high–order bit.  It is just another bit, and not interpreted in any 
special way.  To the physical adder, the high–order is just another bit.

For the adder, we have just the following, assuming that B = – A.

But the 2N represents a carry–out from the high–order (or sign) bit; it is not part of the 
sum, which is still 0.  In order to see this, consider the sum 100 + (–100), considered 
as the sum of two eight–bit two’s–complement integers.

+100 =   0110 0100
	-100 =   1001 1100
	Sum is 1 0000 0000

The eight–bit sum is still 0.

Arithmetic Overflow – “Busting the Arithmetic”

We continue our examination of computer arithmetic to consider one more topic – overflow.  
Arithmetic overflow occurs under a number of cases:

1)	when two positive numbers are added and the result is negative
	2)	when two negative numbers are added and the result is positive
	3)	when a shift operation changes the sign bit of the result.

In mathematics, the sum of two negative numbers is always negative and the sum of two 
positive numbers is always positive.  The overflow problem is an artifact of the limits on the 
range of integers and real numbers as stored in computers.  We shall consider only overflows 
arising from integer addition.

For two’s-complement arithmetic, the range of storable integers is as follows:

16-bit	– 215 to 215 – 1	or – 32768	to 32767

32-bit	– 231 to 231 – 1	or – 2147483648	to 2147483647

In two’s-complement arithmetic, the most significant (left-most) bit is the sign bit

Overflow in addition occurs when two numbers, each with a sign bit of 0, are added and the 
sum has a sign bit of 1 or when two numbers, each with a sign bit of 1, are added and the sum 
has a sign bit of 0.  For simplicity, we consider 16-bit addition.  As an example, consider the 
sum 24576 + 24576 in both decimal and binary.  Note 24576 = 16384 + 8192 = 214 + 213.

24576	0110 0000 0000 0000
	24576	0110 0000 0000 0000
	– 16384	1100 0000 0000 0000

In fact, 24576 + 24576 = 49152 = 32768 + 16384.  The overflow is due to the fact that 49152 
is too large to be represented as a 16-bit signed integer.

As another example, consider the sum (–32768) + (–32768).  As a 16–bit signed integer, 
the sum is 0!

–32768	1000 0000 0000 0000
	–32768	1000 0000 0000 0000
	0	0000 0000 0000 0000

It is easily shown that addition of a validly positive integer to a valid negative integer cannot 
result in an overflow.  For example, consider again 16–bit two’s–complement integer 
arithmetic with two integers M and N.  We have 0  M  32767 and –32768  N  0.  If
|M|  |N|, we have 0  (M + N)  32767 and the sum is valid.  Otherwise, we have 
–32768  (M + N)  0, which again is valid.

Integer overflow can also occur with subtraction.  In this case, the two values (minuend 
and subtrahend) must have opposite signs if overflow is to be possible.

Excess–127

We now cover excess–127 representation.  This is mentioned only because it is required 
when discussing the IEEE floating point standard.  In general, we can consider an excess-M 
notation for any positive integer M.  For an N-bit excess-M representation, the rules for 
conversion from binary to decimal are:

1) Evaluate as an unsigned binary number

2) Subtract M.

To convert from decimal to binary, the rules are

1) Add M

2) Evaluate as an unsigned binary number.

In considering excess notation, we focus on eight-bit excess-127 notation.  The range of 
values that can be stored is based on the range that can be stored in the plain eight-bit 
unsigned standard: 0 through 255.  Remember that in excess-127 notation, to store an integer 
N we first form the number N + 127.  The limits on the unsigned eight-bit storage require 
that 0  (N + 127)  255, or – 127  N  128.

As an exercise, we note the eight-bit excess-127 representation of – 5, – 1, 0 and 4.

– 5 + 127 = 122.	Decimal 122 = 0111 1010 binary, the answer.

– 1 + 127 = 126.	Decimal 126 = 0111 1110 binary, the answer.

0 + 127 = 127.		Decimal 127 = 0111 1111 binary, the answer.

4 + 127 = 131		Decimal 131 = 1000 0011 binary, the answer.

We have now completed the discussion of common ways to represent unsigned and signed 
integers in a binary computer.  We now start our progress towards understanding the storage 
of real numbers in a computer.  There are two ways to store real numbers – fixed point and 
floating point.  We focus this discussion on floating point, specifically the IEEE standard for 
storing floating point numbers in a computer.

Normalized Numbers

The last topic to be discussed prior to defining the IEEE standard for floating point numbers 
is that of normalized numbers.  We must also mention the concept of denormalized numbers, 
though we shall spend much less time on the latter.

A normalized number is one with a representation of the form X  2P, where 1.0  X < 2.0.  
At the moment, we use the term denormalized number to mean a number that cannot be so 
represented, although the term has a different precise meaning in the IEEE standard.  First, 
we ask a question: “What common number cannot be represented in this form?”

The answer is zero.  There is no power of 2 such that 0.0 = X  2P, where 1.0  X < 2.0.  We 
shall return to this issue when we discuss the IEEE standard, at which time we shall give a 
more precise definition of the denormalized numbers, and note that they include 0.0.  For the 
moment, we focus on obtaining the normalized representation of positive real numbers.

We start with some simple examples.

1.0	= 1.0  20, thus X = 1.0 and P = 0.

1.5	= 1.5  20, thus X = 1.5 and P = 0.

2.0	= 1.0  21, thus X = 1.0 and P = 1

0.25	= 1.0  2-2, thus X = 1.0 and P = -2

7.0	= 1.75  22, thus X = 1.75 and P = 2

0.75	= 1.5  2-1, thus X = 1.5 and P = -1.

To better understand this conversion, we shall do a few more examples using the more 
mechanical approach to conversion of decimal numbers to binary.  We start with an 
example: 9.375  10-2 = 0.09375.  We now convert to binary.

0.09375	 2 = 0.1875	0
0.1875	 2 = 0.375	0		Thus decimal 0.09375 = 0.00011 binary
0.375	 2 = 0.75	0		or 1.1  2-4 in the normalized notation.
0.75	 2 = 1.5	1
0.5	 2 = 1.0	1

Please note that these representations take the form X  2P, where X is represented as a 
binary number but P is represented as a decimal number.  Later, P will be converted to an 
excess-127 binary representation, but for the present it is easier to keep it in decimal.

We now convert the decimal number 80.09375 to binary notation.  I have chosen 0.09375 as 
the fractional part out of laziness as we have already obtained its binary representation.  We 
now convert the number 80 from decimal to binary.  Note 80 = 64 + 16 = 26  (1 + ¼).

80 / 2	= 40	remainder 0
40/2	= 20	remainder 0

20 / 2	= 10	remainder 0

10 / 2	= 5	remainder 0

5 / 2	= 2	remainder 1

2 / 2	= 1	remainder 0

1 / 2	= 1	remainder 1

Thus decimal 80 = 1010000 binary and decimal 80.09375 = 1010000.00011 binary.  To get 
the binary point to be after the first 1, we move it six places to the left, so the normalized 
form of the number is 1.01000000011  26, as expected.  For convenience, we write this as 
1.0100 0000 0110  26.

Extended Example: Avagadro’s Number.

Up to this point, we have discussed the normalized representation of positive real numbers 
where the conversion from decimal to binary can be done exactly for both the integer and 
fractional parts.  We now consider conversion of very large real numbers in which it is not 
practical to represent the integer part, much less convert it to binary.

We now discuss a rather large floating point number: 6.023  1023.  This is Avogadro’s 
number.  We shall convert this to normalized form and use the opportunity to discuss a 
number of issues associated with floating point numbers in general.

Avagadro’s number arises in the study of chemistry.  This number relates the atomic weight 
of an element to the number of atoms in that many grams of the element.  The atomic weight 
of oxygen is 16.00, as a result of which there are about 6.023  1023 atoms in 16 grams of 
oxygen.  For our discussion we use a more accurate value of 6.022142  1023 obtained from 
the web sit of the National Institute of Standards (www.nist.gov).

We first remark that the number is determined by experiment, so it is not known exactly.  We 
thus see one of the main scientific uses of this notation – to indicate the precision with which 
the number is known.  The above should be read as (6.022142  0.0000005)  1023, that is to 
say that the best estimate of the value is between 6.0221415  1023 and 6.0221425  1023, or 
between 602, 214, 150, 000, 000, 000, 000, 000 and 602, 214, 250, 000, 000, 000, 000, 000.  
Here we see another use of scientific notation – not having to write all these zeroes.

Again, we use logarithms and anti-logarithms to convert this number to a power of two.  The 
first question is how accurately to state the logarithm.  The answer comes by observing that 
the number we are converting is known to seven digit’s precision.  Thus, the most accuracy 
that makes sense in the logarithm is also seven digits.

In base-10 logarithms log(6.022142  1023) = 23.0 + log(6.022142).  To seven digits, 
this last number is 0.7797510, so log(6.022142  1023) = 23.7797510.

We now use the fact that log(2.0) = 0.3010300 to seven decimal places to solve the equation
	2X = (100.3010300)X = 10.23.7797520 or 0.30103X = 23.7797510 for X = 78.9946218.

If we use NA to denote Avagadro’s number, the first thing we have discovered from this 
tedious analysis is that 278 < NA < 279, and that NA  279.  The representation of the number in 
normal form is thus of the form 1.f  278, where the next step is to determine f.  To do this, 
we obtain the decimal representation of 278.

Note that 278 = (100.30103)78 = 1023.48034 = 100.48034  1023 = 3.022317  1023.
But 6.022142 / 3.022317 = 1.992558, so NA = 1.992558  278, and f = 0.992558.

To complete this problem, we obtain the binary equivalent of 0.992558.

0.992558  2 = 1.985116	1
0.985116  2 = 1.970232	1
0.970232  2 = 1.949464	1
0.949464  2 = 1.880928	1
0.880928  2 = 1.761856	1
0.761856  2 = 1.523712	1
0.523712  2 = 1.047424	1

0.047424  2 = 0.094848	0
0.094848  2 = 0.189696	0
0.189696  2 = 0.379392	0
0.379392  2 = 0.758784	0

0.758784  2 = 1.517568	1

The desired form is 1.1111 1110 0001  278.

# IEEE Standard 754 Floating Point Numbers

There are two primary formats in the IEEE 754 standard; single precision and 
double precision.  We shall study the single precision format.

The single precision format is a 32–bit format.  From left to right, we have
	1 sign bit; 1 for negative and 0 for non-negative
	8 exponent bits
	23 bits for the fractional part of the mantissa.

The eight-bit exponent field stores the exponent of 2 in excess-127 form, with the exception 
of two special bit patterns.

0000 0000	numbers with these exponents are denormalized
	1111 1111	numbers with these exponents are infinity or Not A Number

Before presenting examples of the IEEE 754 standard, we shall examine the concept of 
NaN or Not a Number.  In this discussion, we use some very imprecise terminology.

Consider the quotient 1/0.  The equation 1 / 0 = X is equivalent to solving for a number X 
such that 0  X = 1.  There is no such number.  Loosely speaking, we say 1 / 0 = .  Now 
consider the quotient 0/0.  Again we are asking for the number X such that 0  X = 0.  The 
difference here is that this equation is true for every number X.  In terms of the IEEE 
standard, 0 / 0 is Not a Number, or NaN.

The number NaN can also be used for arithmetic operations that have no solutions, such as 
taking the square root of –1 while limited to the real number system.  While this result 
cannot be represented, it is definitely neither + nor – .

We now illustrate the standard by doing some conversions.

For the first example, consider the number –0.75.

To represent the number in the IEEE standard, first note that it is negative so that the sign bit 
is 1.  Having noted this, we convert the number 0.75.

0.75	 2	= 1.5		1

0.5	 2	= 1.0		1

Thus, the binary equivalent of decimal 0.75 is 0.11 binary.  We must now convert this into 
the normalized form 1.10  2–1.  Thus we have the key elements required.

The power of 2 is –1, stored in Excess-127 as 126 = 0111 1110 binary.

The fractional part is 10, possibly best written as 10000

Recalling that the sign bit is 1, we form the number as follows:

1 0111 1110 10000

We now group the binary bits by fours from the left until we get only 0’s.

1011 1111 0100 0000

Since trailing zeroes are not significant in fractions, this is equivalent to

1011 1111 0100 0000 0000 0000 0000 0000

or BF40 0000 in hexadecimal.

As another example, we revisit a number converted earlier.  We have shown that

80.09375 = 1.0100 0000 0110  26.  This is a positive number, so the sign bit is 0.  As an 
Excess-127 number, 6 is stored as 6 + 127 = 133 = 1000 0101 binary.  The fractional part 
of the number is 0100 0000 0110 0000, so the IEEE representation is

0 1000 0101 0100 0000 0110 0000

Regrouping by fours from the left, we get the following

0100 0010 1010 0000 0011 0000

In hexadecimal this number is 42A030, or 42A0 3000 as an eight digit hexadecimal.

Some Examples “In Reverse”

We now consider another view on the IEEE floating point standard – the “reverse” view.  
We are given a 32-bit number, preferably in hexadecimal form, and asked to produce the 
floating-point number that this hexadecimal string represents.  As always, in interpreting any 
string of binary characters, we must be told what standard to apply – here the IEEE-754 
single precision standard.

First, convert the following 32-bit word, represented by eight hexadecimal digits, to the 
floating-point number being represented.

0000 0000		// Eight hexadecimal zeroes representing 32 binary zeroes

The answer is 0.0.  This is a result that should be memorized.

The question in the following paragraph was taken from a CPSC 2105 mid-term exam and 
the paragraphs following were taken from the answer key for the exam.

Give the value of the real number (in standard decimal representation) represented by the 
following 32-bit words stored as an IEEE standard single precision.
		a)	4068 0000
		b)	42E8 0000
		c)	C2E8 0000
		d)	C380 0000

e)	C5FC 0000

The first step in solving these problems is to convert the hexadecimal to binary.

a)	4068 0000 = 0100 0000 0110 1000 0000 0000 0000 0000
Regroup to get 0  1000 0000  1101 0000 etc.

Thus	s = 0 (not a negative number)
		p + 127 = 100000002 = 12810, so p = 1
and		m = 1101, so 1.m = 1.1101 and the number is 1.110121 = 11.1012.

But 11.1012 = 2 + 1 + 1/2 + 1/8 = 3 + 5/8 = 3.625.

b)	42E8 0000 = 0100 0010 1110 1000 0000 0000 0000 0000
Regroup to get 0  1000 0101  1101 0000 etc
Thus	s = 0 (not a negative number)
		p + 127 = 100001012 = 128 + 4 + 1 = 133, hence p = 6
and		m = 1101, so 1.m = 1.1101 and the number is 1.110126 = 11101002 
But 11101002 = 64 + 32 + 16 + 4 = 96 + 20 = 116 = 116.0

c)	C2E80 0000 = 1100 0010 1110 1000 0000 0000 0000 0000
Regroup to get 1 1000 0101 1101 0000 etc.
Thus	s = 1 (a negative number) and the rest is the same as b).  So – 116.0

d)	C380 0000 = 1100 0011 1000 0000 0000 0000 0000 0000

Regroup to get 1  1000 0111  0000 0000 0000 0000 0000 000

Thus	s = 1 (a negative number)
		p + 127 = 100001112 = 128 + 7 = 135; hence p = 8.
		m = 0000, so 1.m = 1.0 and the number is  – 1.0  28 =  – 256.0

e)	C5FC 0000 = 1100 0101 1111 1100 0000 0000 0000 0000
Regroup to get 1  1000 1011  1111 1000 0000 0000 0000 000
Thus	s = 1 (a negative number)
		p + 127 = 1000 10112 = 128 + 8 + 2 + 1 = 139, so p = 12
		m = 1111 1000, so 1.m = 1.1111 1000

There are three ways to get the magnitude of this number.  The magnitude can be written 
in normalized form as 1.1111 1000  212 = 1.1111 1000  4096, as 212 = 4096.

Method 1

If we solve this the way we have, we have to place four extra zeroes after the decimal point 
to get the required 12, so that we can shift the decimal point right 12 places.

1.1111 1000  212	= 1.1111 1000 0000  212 = 1 1111 1000 00002
				= 212 + 211+ 210 + 29 + 28 + 27
				= 4096 + 2048 + 1024 + 512 + 256 + 128 = 8064.

Method 2

We shift the decimal place only 5 places to the right (reducing the exponent by 5) to get
	1.1111 1000  212	= 1 1111 1.0  27
				= (25 + 24 + 23 + 22 + 21 + 20)  27
				= (32 + 16 + 8 + 4 + 2 + 1)  128 = 63  128 = 8064.

Method 3
This is an offbeat method, not much favored by students.

1.1111 1000  212	= (1 + 2–1 + 2–2+ 2–3+ 2–4+ 2–5)  212
				= 212 + 211+ 210 + 29 + 28 + 27
				= 4096 + 2048 + 1024 + 512 + 256 + 128 = 8064.

Method 4
This is another offbeat method, not much favored by students.

1.1111 1000  212	= (1 + 2–1 + 2–2+ 2–3+ 2–4+ 2–5)  212
				= (1 + 0.5 + 0.25 + 0.125 + 0.0625+ 0.03125)  4096
				= 1.96875  4096 = 8064.

The answer is – 8064.0.

As a final example, we consider the IEEE standard representation of Avogadro’s number. 
We have seen that NA = 1.1111 1110 0001  278.  This is a positive number; the sign bit is 0.

We now consider the representation of the exponent 78.  Now 78 + 127 = 205, so the 
Excess-127 representation of 78 is 205 = 128 + 77 = 128 + 64 + 13 = 128 + 64 + 8 + 4 + 1.  
As an 8-bit binary number this is 1100 1101.  We already have the fractional part, so we get

0 1100 1101 1111 1110 0001 0000

Grouped by fours from the left we get

0110 0110 1111 1111 0000 1000 0000 0000

or 66FF 0800 in hexadecimal.

# Range and Precision

We now consider the range and precision associated with the IEEE single precision standard 
using normalized numbers..  The range refers to the smallest and largest positive numbers 
that can be stored.  Recalling that zero is not a positive number, we derive the smallest and 
largest representable numbers.

In the binary the smallest normalized number is 1.0  2–126 and the largest number is a bit less 
than 2.0  2127 = 2128.  Again, we use logarithms to evaluate these numbers.
	– 126  0.30103 = – 37.93 = – 38.0 + 0.07, so 2–126 = 1.07  10 -38, approximately.

128  0.30103 = 38.53, so 2128 = 3.5  10 38, as 100.53 is a bit bigger than 3.2.

We now consider the precision associated with the standard.  Consider the decimal notation 
1.23.  The precision associated with this is  0.005 as the number really represents a value 
between 1.225 and 1.235 respectively.

The IEEE standard has a 23-bit fraction.  Thus, the precision associated with the standard is 
1 part in 224 or 1 part in 16  220 = 16  1048576 = 16777216.  This accuracy is more precise 
than 1 part in 107, or seven digit precision.

# Denormalized Numbers

We shall see in a bit that the range of normalized numbers is approximately 10 –38 to 10 38.  
We now consider what we might do with a problem such as the quotient 10 –20 / 10 30.  In 
plain algebra, the answer is simply 10 –50, a very small positive number.  But this number is 
smaller than allowed by the standard.  We have two options for representing the quotient, 
either 0.0 or some strange number that clearly indicates the underflow.  This is the purpose of 
denormalized numbers – to show that the result of an operation is positive but too small.

Why Excess–127 Notation for the Exponent?

We have introduced two methods to be used for storing signed integers – two’s-complement 
notation and excess–127 notation.  One might well ask why two’s-complement notation is 
used to store signed integers while the excess–127 method is used for exponents in the 
floating point notation.

The answer for integer notation is simple.  It is much easier to build an adder for integers 
stored in two’s-complement form than it is to build an adder for integers in the excess 
notation.  In the next chapter we shall investigate a two’s-complement adder.

So, why use excess–127 notation for the exponent in the floating point representation?  The 
answer is best given by example.  Consider some of the numbers we have used as examples.

0011 1111 0100 0000 0000 0000 0000 0000 for 0.75

0100 0010 1010 0000 0011 0000 0000 0000 for 80.09375

0110 0110 1111 1111 0000 1000 0000 0000 for Avagadro’s number.

It turns out that the excess–127 notation allows the use of the integer compare unit to 
compare floating point numbers.  Consider two floating point numbers X and Y.  Pretend that 
they are integers and compare their bit patterns as integer bit patterns.  It viewed as an 
integer, X is less than Y, then the floating point number X is less than the floating point Y.  
Note that we are not converting the numbers to integer form, just looking at the bit patterns 
and pretending that they are integers.

Floating Point Equality: X == Y

Due to round off error, it is sometimes not advisable to check directly for equality of floating 
point numbers.  A better method would be to use an acceptable relative error.  We borrow the 
notation  from calculus to stand for a small number, and use the notation |Z| for the absolute 
value of the number Z.

Here are two valid alternatives to the problematic statement (X == Y).

1)	Absolute difference	|X – Y|  

2)	Relative difference	|X – Y|  (|X| +|Y|)

Note that this form of the second statement is preferable to computing the quotient
|X – Y| / (|X| +|Y|) which will be NaN (Not A Number) if X = 0.0 and Y = 0.0.

Bottom Line: In your coding with real numbers, decide what it means for two numbers to be 
equal.  How close is close enough?  There are no general rules here, only cautions.  It is 
interesting to note that one language (SPARK, a variant of the Ada programming language 
does not allow floating point comparison statements such as X == Y, but demands an 
evaluation of the absolute value of the difference between X and Y.

The IBM Mainframe Floating–Point Formats

In this discussion, we shall adopt the bit numbering scheme used in the IBM documentation, 
with the leftmost (sign) bit being number 0.  The IBM Mainframe supports three formats; 
those representations with more bits can be seen to afford more precision.

Single precision	32 bits	numbered 0 through 31,
	Double precision	64 bits	numbered 0 through 63, and
	Extended precision	128 bits	numbered 0 through 127.

As in the IEEE–754 standard, each floating point number in this standard is specified by 
three fields: the sign bit, the exponent, and the fraction.  Unlike the IEEE–754 standard, the 
IBM standard allocates the same number of bits for the exponent of each of its formats.  The 
bit numbers for each of the fields are shown below.

| Format | Sign bit | Bits for exponent | Bits for fraction |
| --- | --- | --- | --- |
| Single precision | 0 | 1 – 7 | 8 – 31 |
| Double precision | 0 | 1 – 7 | 8 – 63 |
| Extended precision | 0 | 1 – 7 | 8 – 127 |

Note that each of the three formats uses eight bits to represent the exponent, in what is 
called the characteristic field, and the sign bit.  These two fields together will be 
represented by two hexadecimal digits in a one–byte field.

The size of the fraction field does depend on the format.
	Single precision	24 bits	6 hexadecimal digits,
	Double precision	56 bits	14 hexadecimal digits, and
	Extended precision	120 bits	30 hexadecimal digits.

The Characteristic Field

In IBM terminology, the field used to store the representation of the exponent is called the 
“characteristic”.  This is a 7–bit field, used to store the exponent in excess–64 format; if the 
exponent is E, then the value (E + 64) is stored as an unsigned 7–bit number.

Recalling that the range for integers stored in 7–bit unsigned format is 0  N  127, we have 
0  (E + 64)  127, or –64  E  63.

Range for the Standard

We now consider the range and precision associated with the IBM floating point formats.  
The reader should remember that the range is identical for all of the three formats; only the 
precision differs.  The range is usually specified as that for positive numbers, from a very 
small positive number to a large positive number.  There is an equivalent range for negative 
numbers.  Recall that 0 is not a positive number, so that it is not included in either range.

Given that the base of the exponent is 16, the range for these IBM formats is impressive.  It is 
from somewhat less than 16–64 to a bit less than 1663.  Note that 1663 = (24)63 = 2252, and 
16–64 = (24)–64 = 2–256 = 1.0 / (2256) and recall that log10(2) = 0.30103.  Using this, we compute 
the maximum number storable at about (100.30103)252 = 1075.86  91075.  We may approximate 
the smallest positive number at 1.0 / (361075) or about 3.010–77.  In summary, the following 
real numbers can be represented in this standard: X = 0.0 and 3.010–77 < X < 91075.

One would not expect numbers outside of this range to appear in any realistic calculation.

Precision for the Standard

Unlike the range, which depends weakly on the format, the precision is very dependent on 
the format used.  More specifically, the precision is a direct function of the number of bits 
used for the fraction.  If the fraction uses F bits, the precision is 1 part in 2F.

We can summarize the precision for each format as follows.
	Single precision	F = 24	1 part in 224.
	Double precision	F = 56	1 part in 256.
	Extended precision	F = 120	1 part in 2120.

The first power of 2 is easily computed; we use logarithms to approximate the others.
	224					= 16,777,216 
	256	 (100.30103)56 = 1016.85		 91016.
	2120	 (100.30103)120 = 1036.12	 1.21036.

The argument for precision is quite simple.  Consider the single precision format, which is 
more precise than 1 part in 10,000,000 and less precise than 1 part in 100,000,000.  In other 
words it is better than 1 part in 107, but not as good as 1 in 108; hence we say 7 digits.

Range and Precision

We now summarize the range and precision for the three IBM Mainframe formats.

| Format | Positive Range | Precision |
| --- | --- | --- |
| Single Precision | 3.010–77 < X < 91075 | 7 digits |
| Double Precision | 3.010–77 < X < 91075 | 16 digits |
| Extended Precision | 3.010–77 < X < 91075 | 36 digits |

Representation of Floating Point Numbers

As with the case of integers, we shall most commonly use hexadecimal notation to represent 
the values of floating–point numbers stored in the memory.  From this point, we shall focus 
on the two more commonly used formats: Single Precision and Double Precision.

The single precision format uses a 32–bit number, represented by 8 hexadecimal digits.
The double precision format uses a 64–bit number, represented by 16 hexadecimal digits.

Due to the fact that the two formats use the same field length for the characteristic, 
conversion between the two is quite simple.  To convert a single precision value to a double 
precision value, just add eight hexadecimal zeroes.

Consider the positive number 128.0.
As a single precision number, the value is stored as	4280 0000.
As a double precision number, the value is stored as	4280 0000 0000 0000.

Conversions from double precision to single precision format will involve some rounding.
For example, consider the representation of the positive decimal number 123.45.  In a few 
pages, we shall show that it is represented as follows.

As a double precision number, the value is stored as	427B 7333 3333 3333.
As a single precision number, the value is stored as	427B 7333.

The Sign Bit and Characteristic Field
We now discuss the first two hexadecimal digits in the representation of a floating–point 
number in these two IBM formats.  In IBM nomenclature, the bits are allocated as follows.
	Bit 0		the sign bit
	Bits 1 – 7	the seven–bit number storing the characteristic.

| Bit Number | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex digit | 0 |  |  |  | 1 |  |  |  |
| Use | Sign bit | Characteristic (Exponent + 64) |  |  |  |  |  |  |

Consider the four bits that comprise hexadecimal digit 0.  The sign bit in the floating–point 
representation is the “8 bit” in that hexadecimal digit.  This leads to a simple rule.

If the number is not negative,	bit 0 is 0, and hex digit 0 is one of 0, 1, 2, 3, 4, 5, 6, or 7.
If the number is negative, 	bit 0 is 1, and hex digit 0 is one of 8, 9, A, B, C, D, E, or F.

Some Single Precision Examples
We now examine a number of examples, using the IBM single–precision floating–point 
format.  The reader will note that the methods for conversion from decimal to hexadecimal 
formats are somewhat informal, and should check previous notes for a more formal method.  
Note that the first step in each conversion is to represent the magnitude of the number in the 
required form X16E, after which we determine the sign and build the first two hex digits.

Example 1: Positive exponent and positive fraction.

The decimal number is 128.50.  The format demands a representation in the form X16E, 
with 0.625  X < 1.0.  As 128  X < 256, the number is converted to the form X162.
Note that 128 = (1/2)162 = (8/16)162 , and 0.5 = (1/512)162 = (8/4096)162.
Hence, the value is 128.50 = (8/16 + 0/256 + 8/4096)162; it is 1620x0.808.

The exponent value is 2, so the characteristic value is either 66 or 0x42 = 100 0010.  The 
first two hexadecimal digits in the eight digit representation are formed as follows.

| Field | Sign | Characteristic |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| Hex value | 4 |  |  |  | 2 |  |  |  |

The fractional part comprises six hexadecimal digits, the first three of which are 808.
The number 128.50 is represented as 4280 8000.

Example 2: Positive exponent and negative fraction.

The decimal number is the negative number –128.50.  At this point, we would normally 
convert the magnitude of the number to hexadecimal representation.  This number has the 
same magnitude as the previous example, so we just copy the answer; it is 1620x0.808.

We now build the first two hexadecimal digits, noting that the sign bit is 1.

| Field | Sign | Characteristic |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| Hex value | C |  |  |  | 2 |  |  |  |

The number 128.50 is represented as C280 8000.
Note that we could have obtained this value just by adding 8 to the first hex digit.

Example 3: Negative exponent and positive fraction.

The decimal number is 0.375.  As a fraction, this is 3/8 = 6/16.  Put another way, it is 
1600.375 = 160(6/16).  This is in the required format X16E, with 0.625  X < 1.0.

The exponent value is 0, so the characteristic value is either 64 or 0x40 = 100 0000.  The first 
two hexadecimal digits in the eight digit representation are formed as follows.

| Field | Sign | Characteristic |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| Hex value | 4 |  |  |  | 0 |  |  |  |

The fractional part comprises six hexadecimal digits, the first of which is a 6.
The number 0.375 is represented in single precision as	4060 0000.
The number 0.375 is represented in double precision as	4060 0000 0000 0000.

Example 4: A Full Conversion
The number to be converted is 123.45.  As we have hinted, this is a non–terminator.

Convert the integer part.
123 / 16 = 7 with remainder 11		this is hexadecimal digit B.
  7 / 16 = 0 with remainder  7		this is hexadecimal digit 7.
Reading bottom to top, the integer part converts as 0x7B.

Convert the fractional part.
0.45  16 = 7.20	Extract the 7,
0.20  16 = 3.20	Extract the 3,
0.20  16 = 3.20	Extract the 3,
0.20  16 = 3.20	Extract the 3, and so on.

In the standard format, this number is 1620x0.7B33333333…...

The exponent value is 2, so the characteristic value is either 66 or 0x42 = 100 0010.  
The first two hexadecimal digits in the eight digit representation are formed as follows.

| Field | Sign | Characteristic |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| Hex value | 4 |  |  |  | 2 |  |  |  |

The number 123.45 is represented in single precision as	427B 3333.
The number 0.375 is represented in double precision as	427B 3333 3333 3333.

Example 5: One in “Reverse”
We are given the single precision representation of the number.  It is 4110 0000.
What is the value of the number stored?  We begin by examination of the first two hex digits.

| Field | Sign | Characteristic |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| Hex value | 4 |  |  |  | 1 |  |  |  |

The sign bit is 0, so the number is positive.  The characteristic is 0x41, so the exponent is
1 and the value may be represented by X161.  The fraction field is 100 000, so the value is
161(1/16) = 1.0.

Packed Decimal Formats

While the IBM mainframe provides three floating–point formats, it also provides another 
format for use in what are called “fixed point” calculations.  The term “fixed point” refers to 
decimal numbers in which the decimal point takes a predictable place in the number; money 
transactions in dollars and cents are a good and very important example of this.

Consider a ledger such as might be maintained by a commercial firm.  This contains credits 
and debits, normally entered as money amounts with dollars and cents.  The amount that 
might be printed as “$1234.56” could easily be stored as the integer 123456 if the program 
automatically adjusted to provide the implicit decimal point.  This fact is the basis for the 
Packed Decimal Format developed by IBM in response to its business customers.

One may well ask “Why not use floating point formats for financial transactions?”.  We 
present a fairly realistic scenario to illustrate the problem with such a choice.  This example 
is based on your author’s experience as a consultant to a bank in Rochester, NY.

It is a fact that banks loan each other money on an overnight basis; that is, the bank borrows 
the money at 6:00 PM today and repays it at 6:00 AM tomorrow.  While this may seem a bit 
strange to those of us who think in terms of 20–year mortgages, it is an important practice.

Overnight loans in the amount of one hundred million dollars are not uncommon.

Suppose that I am a bank officer, and that another bank wants to borrow $100,000,000 
overnight.  I would like to make the loan, but do not have the cash on hand.  On the other 
hand, I know a bank that will lend me the money at a smaller interest rate.  I can make the 
loan and pocket the profit.

Suppose that the borrowing bank is willing to pay 8% per year on the borrowed amount.  
This corresponds to a payback of (1.08)1/730 = 1.0001054, which is $10,543 in interest.

Suppose that I have to borrow the money at 6% per annum.  This corresponds to my paying 
at a rate of (1.06)1/730 = 1.0000798, which is a cost of $7,982 to me.  I make $2,561.

Consider these numbers as single–precision floating point format in the IBM Mainframe.

My original money amount is	$100,000,000

The interest I make is	$10,543

My principal plus interest is	$100,010,500	Note the truncation due to precision.
The interest I pay is	$7,982
What I get back is	$100,002,000	Again, note the truncation.

The use of floating–point arithmetic has cost me $561 for an overnight transaction.  I do not 
like that.  I do not like numbers that are rounded off; I want precise arithmetic.

Almost all banks and financial institutions demanded some sort of precise decimal 
arithmetic; IBM’s answer was the Packed Decimal format.

BCD (Binary Coded Decimal)

The best way to introduce the Packed Decimal Data format is to first present an earlier 
format for encoding decimal digits. This format is called BCD, for “Binary Coded Decimal”.  
As may be inferred from its name, it is a precursor to EBCDIC (Extended BCD Interchange 
Code) in addition to heavily influencing the Packed Decimal Data format.

We shall introduce BCD and compare it to the 8–bit unsigned binary previously discussed for 
storing unsigned integers in the range 0 through 255 inclusive.  While BCD doubtless had 
encodings for negative numbers, we shall postpone signed notation to Packed Decimal.

The essential difference between BCD and 8–bit binary is that BCD encodes each decimal in 
a separate 4–bit field (sometimes called “nibble” for half–byte).  This contrasts with the usual 
binary notation in which it is the magnitude of the number, and not the number of digits, that 
determines whether or not it can be represented in the format.

We begin with a table of the BCD codes for each of the ten decimal digits.  These codes are 
given in both binary and hexadecimal.  It will be important for future discussions to note that 
these encodings are actually hexadecimal digits; they just appear to be decimal digits.

| Digit | ‘0’ | ‘1’ | ‘2’ | ‘3’ | ‘4’ | ‘5’ | ‘6’ | ‘7’ | ‘8’ | ‘9’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 |
| Hexadecimal | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

To emphasize the difference between 8–bit unsigned binary and BCD, we shall examine a 
selection of two–digit numbers and their encodings in each system.

| Decimal Number | 8–bit binary | BCD (Represented in binary) | BCD (hexadecimal) |
| --- | --- | --- | --- |
| 5 | 0000 0101 | 0000 0101 | 05 |
| 13 | 0000 1101 | 0001 0011 | 13 |
| 17 | 0001 0001 | 0001 0111 | 17 |
| 23 | 0001 0111 | 0010 0011 | 23 |
| 31 | 0001 1111 | 0011 0001 | 31 |
| 64 | 0100 0000 | 0110 0100 | 64 |
| 89 | 0101 1001 | 1000 1001 | 89 |
| 96 | 0110 0000 | 1001 0110 | 96 |

As a hypothetical aside, consider the storage of BCD numbers on a byte–addressable 
computer.  The smallest addressable unit would be an 8–bit byte.  As a result of this, all BCD 
numbers would need to have an even number of digits, as to fill up an integral number of 
bytes.  Our solution to the storage of integers with an odd number of digits is to recall that a 
leading zero does not change the value of the integer.

In this hypothetical scheme of storage:
	1	would be stored as	01,
	22	would be stored as	22,
	333	would be stored as	0333,
	4444	would be stored as	4444,
	55555	would be stored as	055555,	and
	666666	would be stored as	666666.

Packed Decimal Data

The packed decimal format should be viewed as a generalization of the BCD format with the 
specific goal of handling the fixed point arithmetic so common in financial transactions.  The 
two extensions of the BCD format are as follows:
	1.	The provision of a sign “half byte” so that negative numbers can be handled.
	2.	The provision for variable length strings.

While the term “fixed point” is rarely used in computer literature these days, the format is 
very common.  Consider any transaction denominated in dollars and cents.  The amount will 
be represented as a real number with exactly two digits to the right of the decimal point; that 
decimal point has a fixed position in the notation, hence the name “fixed point”.

The packed decimal format provides for a varying number of digits, one per half–byte, 
followed by a half–byte denoting the sign of the number.  Because of the standard byte 
addressability issues, the number of half–bytes in the representation must be an even number; 
given the one half–byte reserved for the sign, this implies an odd number of digits.

In the BCD encodings, we use one hexadecimal digit to encode each of the decimal digits.  
This leaves the six higher–valued hexadecimal digits (A, B, C, D, E, and F) with no use; in 
BCD these just do not encode any values.  In Packed Decimal, each of these will encode a 
sign.  Here are the most common hexadecimal digits used to represent signs.

| Binary | Hexadecimal | Sign | Comment |
| --- | --- | --- | --- |
| 1100 | C | + | The standard plus sign |
| 1101 | D | – | The standard minus sign |
| 1111 | F | + | A plus sign seen in converted EBCDIC |

We now move to the IBM implementation of the packed decimal format.  This section breaks 
with the tone previously established in this chapter – that of discussing a format in general 
terms and only then discussing the IBM implementation.  The reason for this change is 
simple; the IBM implementation of the packed decimal format is the only one used.

The Syntax of Packed Decimal Format

1.	The length of a packed decimal number may be from 1 to 31 digits; the
		number being stored in memory as 1 to 16 bytes.

2.	The rightmost half–byte of the number contains the sign indicator.  In constants
		defined by code, this is 0xC for positive numbers and 0xD for negative.

3.	The remaining number of half–bytes (always an odd number) contain the
		hexadecimal encodings of the decimal digits in the number.

4.	The rightmost byte in the memory representation of the number holds one
		digit and the sign half–byte.  All other bytes hold two digits.

5.	The number zero is always represented as the two digits 0C, never 0D.

6.	Any number with an even number of digits will be converted to an equivalent
		number with a prepended “0” prior to storage as packed decimal.

7.	Although the format allows for storage of numbers with decimal points, neither
		the decimal point nor any indication of its position is stored.  As an example, 
		each of 1234.5, 123.45, 12.345, and 1.2345 is stored as 12345C.

There are two common ways to generate numbers in packed decimal format, and quite a 
variety of instructions to operate on data in this format.  We shall discuss these in later 
chapters.  For the present, we shall just show a few examples.

1.	Store the positive number 144 in packed decimal format.

Note that the number 144 has an odd number of digits.  The format just adds the half–byte 
for non–negative numbers, generating the representation 144C.  This value is often written 
as 14 4C, with the space used to emphasize the grouping of half–bytes by twos.

2.	Store the negative number –1023 in packed decimal format.

Note that the magnitude of the number (1023) has an even number of digits, so the format 
will prepend a “0” to produce the equivalent number 01023, which has an odd number of 
digits.  The value stored is 01023D, often written as 01 02 3D.

2.	Store the negative number –7 in packed decimal format.

Note that the magnitude of the number (7) has an odd number of digits, so the format 
just adds the sign half–byte to generate the representation 7D.

4.	Store the positive number 123.456 in packed decimal format.

Note that the decimal point is not stored.  This is the same as the storage of the number
123456 (which has a decidedly different value).  This number has an even number of digits, 
so that it is converted to the equivalent value 0123456 and stored as 01 23 45 6C.

5.	Store the positive number 1.23456 in packed decimal format.

Note that the decimal point is not stored.  This is the same as the storage of the number
123456 (which has a decidedly different value).  This number has an even number of digits, 
so that it is converted to the equivalent value 0123456 and stored as 01 23 45 6C.

6.	Store the positive number 12345.6 in packed decimal format.

Note that the decimal point is not stored.  This is the same as the storage of the number
123456 (which has a decidedly different value).  This number has an even number of digits, 
so that it is converted to the equivalent value 0123456 and stored as 01 23 45 6C.

7.	Store the number 0 in packed decimal form.

Note that 0 is neither positive nor negative.  IBM convention treats the zero as a positive 
number, and always stores it as 0C.

8.	Store the number 12345678901234567890 in packed decimal form.

Note that very large numbers are easily stored in this format.  The number has 20 digits, an 
even number, so it must first be converted to the equivalent 012345678901234567890.  It is 
stored as 01 23 45 67 89 01 23 45 67 89 0C.

Comparison: Floating–Point and Packed Decimal

Here are a few obvious comments on the relative advantages of each format.

1.	Packed decimal format can provide great precision and range, more that is required
	for any conceivable financial transaction.  It does not suffer from round–off errors.

2.	The packed decimal format requires the code to track the decimal points explicitly.
	This is easily done for addition and subtraction, but harder for other operations.
	The floating–point format provides automatic management of the decimal point.

# Character Codes: ASCII

We now consider the methods by which computers store character data.  There are three 
character codes of interest: ASCII, EBCDIC, and Unicode.  The EBCDIC code is only used 
by IBM in its mainframe computer.  The ASCII code is by far more popular, so we consider 
it first and then consider Unicode, which can be viewed as a generalization of ASCII.

The figure below shows the ASCII code.  Only the first 128 characters (Codes 00 – 7F in 
hexadecimal) are standard.  There are several interesting facts.

| Last Digit \ First Digit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | NUL | DLE | SP | 0 | @ | P | ` | p |
| 1 | SOH | DC1 | ! | 1 | A | Q | a | q |
| 2 | STX | DC2 | “ | 2 | B | R | b | r |
| 3 | ETX | DC3 | # | 3 | C | S | c | s |
| 4 | EOT | DC4 | $ | 4 | D | T | d | t |
| 5 | ENQ | NAK | % | 5 | E | U | e | u |
| 6 | ACK | SYN | & | 6 | F | V | f | v |
| 7 | BEL | ETB | ` | 7 | G | W | g | w |
| 8 | BS | CAN | ( | 8 | H | X | h | x |
| 9 | HT | EM | ) | 9 | I | Y | i | y |
| A | LF | SUB | * | : | J | Z | j | z |
| B | VT | ESC | + | ; | K | [ | k | { |
| C | FF | FS | ‘ | < | L | \ | l | ¦ |
| D | CR | GS | - | = | M | ] | m | } |
| E | SO | RS | . | > | N | ^ | n | ~ |
| F | SI | US | / | ? | O | _ | o | DEL |

As ASCII is designed to be an 8–bit code, several manufacturers have defined extended code 
sets, which make use of the codes 0x80 through 0xFF (128 through 255).  One of the more 
popular was defined by IBM.  None are standard; most books ignore them.  So do we.

Let X be the ASCII code for a digit.  Then X – ‘0’ = X – 30 is the value of the digit.
For example ASCII(‘7’) = 37, with value 37 – 30 = 7.

Let X be an ASCII code.	If ASCII(‘A’)  X  ASCII(‘Z’) then X is an upper case letter.
	If ASCII(‘a’)  X  ASCII(‘z’) then X is an lower case letter.

If ASCII(‘0’)  X  ASCII(‘9’) then X is a decimal digit.

Let X be an upper-case letter.  Then ASCII ( lower_case(X) ) = ASCII ( X ) + 32

Let X be a lower case letter.  Then ASCII ( UPPER_CASE(X) ) = ASCII (X ) – 32.
The expressions are ASCII ( X ) + 20 and ASCII (X ) – 20 in hexadecimal.

Character Codes: EBCDIC

The Extended Binary Coded Decimal Interchange Code (EBCDIC) was developed by IBM 
in the early 1960’s for use on its System/360 family of computers.  It evolved from an older 
character set called BCDIC, hence its name.

EBCDIC code uses eight binary bits to encode a character set; it can encode 256 characters.  
The codes are binary numeric values, traditionally represented as two hexadecimal digits.

Character codes 0x00 through 0x3F and 0xFF represent control characters.
	0x0D	is the code for a carriage return; this moves the cursor back to the left margin.
	0x20	is used by the ED (Edit) instruction to represent a packed digit to be printed.
	0x21	is used by the ED (Edit) instruction to force significance.
		All digits, including leading 0’s, from this position will be printed.
	0x25	is the code for a line feed; this moves the cursor down but not horizontally.
	0x2F	is the BELL code; it causes the terminal to emit a “beep”.

Character codes 0x40 through 0x7F represent punctuation characters.
	0x40	is the code for a space character: “ ”.
	0x4B	is the code for a decimal point: “.”.
	0x4E	is the code for a plus sign: “+”.
	0x50	is the code for an ampersand: “&”.
	0x5B	is the code for a dollar sign: “$”.
	0x5C	is the code for an asterisk: “*”.
	0x60	is the code for a minus sign: “–”.
	0x6B	is the code for a comma: “,”.
	0x6F	is the code for a question mark: “?”.
	0x7C	is the code for the commercial at sign: “@”.

Character codes 0x81 through 0xA9 represent the lower case Latin alphabet.
	0x81 through 0x89	represent the letters “a” through “i”,
	0x91 through 0x99	represent the letters “j” through “r”, and
	0xA2 through 0xA9	represent the letters “s” through “z”.

Character codes 0xC1 through 0xE9 represent the upper case Latin alphabet.
	0xC1 through 0xC9	represent the letters “A” through “I”,
	0xD1 through 0xD9	represent the letters “J” through “R”, and
	0xE2 through 0xE9	represent the letters “S” through “Z”.

Character codes 0xF0 through 0xF9 represent the digits “0” through “9”.

NOTES:

1.	The control characters are mostly used for network data transmissions.
		The ones listed above appear frequently in user code for terminal I/O.

2.	There are gaps in the codes for the alphabetical characters.
		This is due to the origins of the codes for the upper case alphabetic characters
		in the card codes used on the IBM–029 card punch.

3.	One standard way to convert an EBCDIC digit to its numeric value
		is to subtract the hexadecimal number 0xF0 from the character code.

An Abbreviated Table: The Common EBCDIC

| Code | Char. | Comment | Code | Char. | Comment | Code | Char. | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 80 |  |  | C0 | } | Right brace |
|  |  |  | 81 | a |  | C1 | A |  |
|  |  |  | 82 | b |  | C2 | B |  |
|  |  |  | 83 | c |  | C3 | C |  |
|  |  |  | 84 | d |  | C4 | D |  |
|  |  |  | 85 | e |  | C5 | E |  |
|  |  |  | 86 | f |  | C6 | F |  |
|  |  |  | 87 | g |  | C7 | G |  |
| 0C | FF | Form feed | 88 | h |  | C8 | H |  |
| 0D | CR | Return | 89 | i |  | C9 | I |  |
| 16 | BS | Back space | 90 |  |  | D0 | { | Left brace |
| 25 | LF | Line Feed | 91 | j |  | D1 | J |  |
| 27 | ESC | Escape | 92 | k |  | D2 | K |  |
| 2F | BEL | Bell | 93 | l |  | D3 | L |  |
| 40 | SP | Space | 94 | m |  | D4 | M |  |
| 4B | . | Decimal | 95 | n |  | D5 | N |  |
| 4C | < |  | 96 | o |  | D6 | O |  |
| 4D | ( |  | 97 | p |  | D7 | P |  |
| 4E | + |  | 98 | q |  | D8 | Q |  |
| 4F | ¦ | Single Bar | 99 | r |  | D9 | R |  |
| 50 | & |  | A0 |  |  | E0 | \ | Back slash |
| 5A | ! |  | A1 | ~ | Tilde | E1 |  |  |
| 5B | $ |  | A2 | s |  | E2 | S |  |
| 5C | * |  | A3 | t |  | E3 | T |  |
| 5D | ) |  | A4 | u |  | E4 | U |  |
| 5E | ; |  | A5 | v |  | E5 | V |  |
| 5F |  | Not | A6 | w |  | E6 | W |  |
| 60 | – | Minus | A7 | x |  | E7 | X |  |
| 61 | / | Slash | A8 | y |  | E8 | Y |  |
| 6A | ¦ | Dbl. Bar | A9 | z |  | E9 | Z |  |
| 6B | , | Comma | B0 | ^ | Carat | F0 | 0 |  |
| 6C | % | Percent | B1 |  |  | F1 | 1 |  |
| 6D | _ | Underscore | B2 |  |  | F2 | 2 |  |
| 6E | > |  | B3 |  |  | F3 | 3 |  |
| 6F | ? |  | B4 |  |  | F4 | 4 |  |
| 79 | ‘ | Apostrophe | B5 |  |  | F5 | 5 |  |
| 7A | : | Colon | B6 |  |  | F6 | 6 |  |
| 7B | # | Sharp | B7 |  |  | F7 | 7 |  |
| 7C | @ | At Sign | B8 |  |  | F8 | 8 |  |
| 7D | ' | Apostrophe | B9 |  |  | F9 | 9 |  |
| 7E | = | Equals | BA | [ | Left Bracket |  |  |  |
| 7F | " | Quote | BB | ] | R. Bracket |  |  |  |

It is worth noting that IBM seriously considered adoption of ASCII as its method for internal 
storage of character data for the System/360.  The American Standard Code for Information 
Interchange was approved in 1963 and supported by IBM.  However the ASCII code set was 
not compatible with the BCDIC used on a very large installed base of support equipment, 
such as the IBM 026.  Transition to an incompatible character set would have required any 
adopter of the new IBM System/360 to also purchase or lease an entirely new set of 
peripheral equipment; this would have been a deterrence to early adoption.

The figure below shows a standard 80–column IBM punch card produced by the IBM 029 
card punch.  This shows the card punch codes used to represent some EBCDIC characters.

The structure of the EBCDIC, used for internal character storage on the System/360 and later 
computers, was determined by the requirement for easy translation from punch card codes.  
The table below gives a comparison of the two coding schemes.

| Remember that the punch card codes represent the card rows punched. Each digit was represented by a punch in a single row; the row number was identical to the value of the digit being encoded. Character | Punch Code | EBCDIC |
| --- | --- | --- |
| ‘0’ | 0 | F0 |
| ‘1’ | 1 | F1 |
| ‘9’ | 9 | F9 |
| ‘A’ | 12 – 1 | C1 |
| ‘B’ | 12 – 2 | C2 |
| ‘I’ | 12 – 9 | C9 |
| ‘J’ | 11 – 1 | D1 |
| ‘K’ | 11 – 2 | D2 |
| ‘R’ | 11 – 9 | D9 |
| ‘S’ | 0 – 2 | E2 |
| ‘T’ | 0 – 3 | E3 |
| ‘Z’ | 0 – 9 | E9 |

The EBCDIC codes are eight–bit binary 
numbers, almost always represented as 
two hexadecimal digits.  Some IBM 
documentation refers to these digits as:
	The first digit is the zone potion,
	The second digit is the numeric.

A comparison of the older card punch codes with the EBCDIC shows that its design was 
intended to facilitate the translation.  For digits, the numeric punch row became the numeric 
part of the EBCDIC representation, and the zone part was set to hexadecimal F.  For the 
alphabetical characters, the second numeric row would become the numeric part and the first 
punch row would determine the zone portion of the EBCDIC.

This matching with punched card codes explains the “gaps” found in the EBCDIC set.  
Remember that these codes are given as hexadecimal numbers, so that the code immediately 
following C9 would be CA (as hexadecimal A is decimal 10).  But the code for ‘J’ is not 
hexadecimal CA, but hexadecimal D1.  Also, note that the EBCDIC representation for the 
letter ‘S’ is not E1 but E2.  This is a direct consequence of the design of the punch cards.

Character Codes: UNICODE

The UNICODE character set is a generalization of the ASCII character set to allow for the 
fact that many languages in the world do not use the Latin alphabet.  The important thing to 
note here is that UNICODE characters consume 16 bits (two bytes) while ASCII and 
EBCDIC character codes are 8 bits (one byte) long.  This has some implications in 
programming with modern languages, such as Visual Basic and Visual C++, especially in 
allocation of memory space to hold strings.  This seems to be less of an issue in Java.

An obvious implication of the above is that, while each of ASCII and EBCDIC use two 
hexadecimal digits to encode a character, UNICODE uses four hexadecimal digits.  In part, 
UNICODE was designed as a replacement for the ad–hoc “code pages” then in use.  These 
pages allowed arbitrary 256–character sets by a complete redefinition of ASCII, but were 
limited to 256 characters.  Some languages, such as Chinese, require many more characters.

UNICODE is downward compatible with the ASCII code set; the characters represented by 
the UNICODE codes 0x0000 through 0x007F are exactly those codes represented by the 
standard ASCII codes 0x00 through 0x7F.  In other words, to convert standard ASCII to 
correct UNICODE, just add two leading hexadecimal 0’s and make a two–byte code.

The origins of Unicode date back to 1987 when Joe Becker from Xerox and Lee Collins and 
Mark Davis from Apple started investigating the practicalities of creating a universal 
character set. In August of the following year Joe Becker published a draft proposal for an 
"international/multilingual text character encoding system, tentatively called Unicode." In 
this document, entitled Unicode 88, he outlined a 16 bit character model:

“Unicode is intended to address the need for a workable, reliable world text 
encoding. Unicode could be roughly described as "wide-body ASCII" that has 
been stretched to 16 bits to encompass the characters of all the world's living 
languages. In a properly engineered design, 16 bits per character are more than 
sufficient for this purpose.”

In fact the 16–bit (four hexadecimal digit) code scheme has proven not to be adequate to 
encode every possible character set.  The original code space (0x0000 – 0xFFFF) was 
defined as the “Basic Multilingual Plane”, or BMP.  Supplementary planes have been 
added, so that as of September 2008 there were over 1,100,000 “code points” in UNICODE.

Here is a complete listing of the character sets and languages supported by the Basic 
Multilingual Plane.  The source is http://www.ssec.wisc.edu/~tomw/java/unicode.html.

| Range | Decimal | Name |
| --- | --- | --- |
| 0x0000-0x007F | 0-127 | Basic Latin |
| 0x0080-0x00FF | 128-255 | Latin-1 Supplement |
| 0x0100-0x017F | 256-383 | Latin Extended-A |
| 0x0180-0x024F | 384-591 | Latin Extended-B |
| 0x0250-0x02AF | 592-687 | IPA Extensions |
| 0x02B0-0x02FF | 688-767 | Spacing Modifier Letters |
| 0x0300-0x036F | 768-879 | Combining Diacritical Marks |
| 0x0370-0x03FF | 880-1023 | Greek |
| 0x0400-0x04FF | 1024-1279 | Cyrillic |
| 0x0530-0x058F | 1328-1423 | Armenian |
| 0x0590-0x05FF | 1424-1535 | Hebrew |
| 0x0600-0x06FF | 1536-1791 | Arabic |
| 0x0700-0x074F | 1792-1871 | Syriac |
| 0x0780-0x07BF | 1920-1983 | Thaana |
| 0x0900-0x097F | 2304-2431 | Devanagari |
| 0x0980-0x09FF | 2432-2559 | Bengali |
| 0x0A00-0x0A7F | 2560-2687 | Gurmukhi |
| 0x0A80-0x0AFF | 2688-2815 | Gujarati |
| 0x0B00-0x0B7F | 2816-2943 | Oriya |
| 0x0B80-0x0BFF | 2944-3071 | Tamil |
| 0x0C00-0x0C7F | 3072-3199 | Telugu |
| 0x0C80-0x0CFF | 3200-3327 | Kannada |
| 0x0D00-0x0D7F | 3328-3455 | Malayalam |
| 0x0D80-0x0DFF | 3456-3583 | Sinhala |
| 0x0E00-0x0E7F | 3584-3711 | Thai |
| 0x0E80-0x0EFF | 3712-3839 | Lao |
| 0x0F00-0x0FFF | 3840-4095 | Tibetan |
| 0x1000-0x109F | 4096-4255 | Myanmar |
| 0x10A0-0x10FF | 4256-4351 | Georgian |
| 0x1100-0x11FF | 4352-4607 | Hangul Jamo |
| 0x1200-0x137F | 4608-4991 | Ethiopic |
| 0x13A0-0x13FF | 5024-5119 | Cherokee |
| 0x1400-0x167F | 5120-5759 | Unified Canadian Aboriginal Syllabics |
| 0x1680-0x169F | 5760-5791 | Ogham |
| 0x16A0-0x16FF | 5792-5887 | Runic |
| 0x1780-0x17FF | 6016-6143 | Khmer |
| 0x1800-0x18AF | 6144-6319 | Mongolian |
| Range | Decimal | Name |
| 0x1E00-0x1EFF | 7680-7935 | Latin Extended Additional |
| 0x1F00-0x1FFF | 7936-8191 | Greek Extended |
| 0x2000-0x206F | 8192-8303 | General Punctuation |
| 0x2070-0x209F | 8304-8351 | Superscripts and Subscripts |
| 0x20A0-0x20CF | 8352-8399 | Currency Symbols |
| 0x20D0-0x20FF | 8400-8447 | Combining Marks for Symbols |
| 0x2100-0x214F | 8448-8527 | Letterlike Symbols |
| 0x2150-0x218F | 8528-8591 | Number Forms |
| 0x2190-0x21FF | 8592-8703 | Arrows |
| 0x2200-0x22FF | 8704-8959 | Mathematical Operators |
| 0x2300-0x23FF | 8960-9215 | Miscellaneous Technical |
| 0x2400-0x243F | 9216-9279 | Control Pictures |
| 0x2440-0x245F | 9280-9311 | Optical Character Recognition |
| 0x2460-0x24FF | 9312-9471 | Enclosed Alphanumerics |
| 0x2500-0x257F | 9472-9599 | Box Drawing |
| 0x2580-0x259F | 9600-9631 | Block Elements |
| 0x25A0-0x25FF | 9632-9727 | Geometric Shapes |
| 0x2600-0x26FF | 9728-9983 | Miscellaneous Symbols |
| 0x2700-0x27BF | 9984-10175 | Dingbats |
| 0x2800-0x28FF | 10240-10495 | Braille Patterns |
| 0x2E80-0x2EFF | 11904-12031 | CJK Radicals Supplement |
| 0x2F00-0x2FDF | 12032-12255 | Kangxi Radicals |
| 0x2FF0-0x2FFF | 12272-12287 | Ideographic Description Characters |
| 0x3000-0x303F | 12288-12351 | CJK Symbols and Punctuation |
| 0x3040-0x309F | 12352-12447 | Hiragana |
| 0x30A0-0x30FF | 12448-12543 | Katakana |
| 0x3100-0x312F | 12544-12591 | Bopomofo |
| 0x3130-0x318F | 12592-12687 | Hangul Compatibility Jamo |
| 0x3190-0x319F | 12688-12703 | Kanbun |
| 0x31A0-0x31BF | 12704-12735 | Bopomofo Extended |
| 0x3200-0x32FF | 12800-13055 | Enclosed CJK Letters and Months |
| 0x3300-0x33FF | 13056-13311 | CJK Compatibility |
| 0x3400-0x4DB5 | 13312-19893 | CJK Unified Ideographs Extension A |
| 0x4E00-0x9FFF | 19968-40959 | CJK Unified Ideographs |
| 0xA000-0xA48F | 40960-42127 | Yi Syllables |
| 0xA490-0xA4CF | 42128-42191 | Yi Radicals |
| 0xAC00-0xD7A3 | 44032-55203 | Hangul Syllables |
| 0xD800-0xDB7F | 55296-56191 | High Surrogates |
| 0xDB80-0xDBFF | 56192-56319 | High Private Use Surrogates |
| Range | Decimal | Name |
| 0xDC00-0xDFFF | 56320-57343 | Low Surrogates |
| 0xE000-0xF8FF | 57344-63743 | Private Use |
| 0xF900-0xFAFF | 63744-64255 | CJK Compatibility Ideographs |
| 0xFB00-0xFB4F | 64256-64335 | Alphabetic Presentation Forms |
| 0xFB50-0xFDFF | 64336-65023 | Arabic Presentation Forms-A |
| 0xFE20-0xFE2F | 65056-65071 | Combining Half Marks |
| 0xFE30-0xFE4F | 65072-65103 | CJK Compatibility Forms |
| 0xFE50-0xFE6F | 65104-65135 | Small Form Variants |
| 0xFE70-0xFEFE | 65136-65278 | Arabic Presentation Forms-B |
| 0xFEFF-0xFEFF | 65279-65279 | Specials |
| 0xFF00-0xFFEF | 65280-65519 | Halfwidth and Fullwidth Forms |
| 0xFFF0-0xFFFD | 65520-65533 | Specials |

Here is a bit of the Greek alphabet as encoded in the BMP.

For those with more esoteric tastes, here is a small sample of Cuneiform in 32–bit Unicode.

Now we see some Egyptian hieroglyphics, also with the 32–bit Unicode encoding.

We close this chapter with a small sample of the 16–bit BMP encoding for the CJK 
(Chinese, Japanese, & Korean) character set.  Unlike the above two examples (Cuneiform 
and Egyptian hieroglyphics) this is a living language.

As a final note, we mention the fact that some fans of the Star Trek series have proposed that 
the alphabet for the Klingon language be included in the Unicode 32–bit encodings.  So far, 
they have inserted it in the Private Use section (0xE000-0xF8FF).  It is not yet recognized as 
an official part of the Unicode standard.

Solved Problems

1.	What range of integers can be stored in an 16–bit word if

a) the number is stored as an unsigned integer?

b) the number is stored in two’s–complement form?

Answer:	a)	0 through 65,535 inclusive, or 0 through 216 – 1.
		b) –32768 through 32767 inclusive, or –(215) through (215) – 1

2	You are given the 16–bit value, represented as four hexadecimal digits,
	and stored in two bytes.  The value is 0x812D.

a)	What is the decimal value stored here, if interpreted as a packed decimal number?

b)	What is the decimal value stored, if interpreted as a 16–bit two’s–complement 
		integer?

c)	What is the decimal value stored here, if interpreted as a 16–bit unsigned integer?

ANSWER:	The answers are found in the lectures for January 13 and January 20.

a)	For a packed decimal number, the absolute value is 812 and the value is negative.
		The answer is –812.

b)	To render this as a two’s–complement integer, one first has to convert to binary.
		Hexadecimal 812D converts to	1000 0001 0010 1101.  This is negative.
		Take the one’s complement to get	0111 1110 1101 0010.
		Add 1 to get the positive value	0111 1110 1101 0011.
		In hexadecimal, this is 7ED3, which converts to 7163 + 14162 + 1316 + 3, 
		or 74096 + 14256 + 1316 + 3 = 28672 + 3584 + 208 + 3 = 32,467
		The answer is –32,467.

c)	As an unsigned binary number the value is obtained by direct conversion from 
		the hexadecimal value.  The value is 8163 + 1162 + 216 + 13, 
		or 84096 + 1256 + 216 + 13 = 32768 + 256 + 32 + 13 = 33069.

3.	Give the 8–bit two’s complement representation of the number – 98.

Answer:	98 = 96 + 2 = 64 + 32 + 2, so its binary representation is 0110 0010.
		8–bit representation of + 98	0110 0010
		One’s complement			1001 1101
		Add 1 to get				1001 1110	9E.

4.	Give the 16–bit two’s complement representation of the number – 98.

Answer:	The 8–bit representation of – 98 is	1001 1110
		Sign extend to 16 bits	1111 1111 1001 1110

5	Convert the following decimal numbers to binary.

a)  37.375	b)  93.40625

ANSWER:  Recall that the integer part and fractional part are converted separately.

a)  37.375
		37 / 2	= 18	rem 1
		18 / 2	=  9	rem 0
		9 / 2	=  4	rem 1
		4 / 2	= 2	rem 0
		2 / 2	= 1	rem 0
		1 / 2	= 0	rem 1	Answer: 100101.

0.375  2	= 0.75
		0.75  2	=  1.50
		0.50  2	= 1.00
		0.00  2	= 0.00		Answer 0.011		100101.011

b)	93.40625
		93 / 2	= 46	rem 1
		46 / 2	= 23	rem 0
		23 / 2	= 11	rem 1
		11 / 2	=  5	rem 1
		5 / 2	=  2	rem 1
		2 / 2	=  1	rem 0
		1 / 2	=  0	rem 1	Answer: 1011101

0.40625  2	= 0.8125
		0.8125  2	= 1.6250
		0.625  2	= 1.2500
		0.25  2	= 0.5000
		0.50  2	= 1.0000
		0.00  2	= 0.0000		Answer: 0.01101	1011101.01101

6	Convert the following hexadecimal number to decimal numbers.
	The numbers are unsigned.  Use as many digits as necessary
	a) 0x022	b) 0x0BAD	c) 0x0EF

ANSWER: A = 10, B = 11, C = 12, D = 13, E = 14, F = 15
	160 = 1,  161 = 16,  162 = 256,  163 = 4096,  164 = 65536
	a)	0x022	= 216 + 2 = 32 + 2	= 34	34
	b)	0x0BAD	= 11162 + 1016 + 13	= 11256 + 1016 + 13
		= 2816 + 160 + 13	= 2989	2989
	c)	0x0EF	= 1416 + 15	= 224 + 15 = 239	239

7	Show the IEEE–754 single precision representation of the following real numbers.
	Show all eight hexadecimal digits associated with each representation.
	a)	0.0	b) – 1.0	c) 7.625	d) – 8.75

ANSWER:
a)	0.0 = 0x0000 0000			0x0000 0000

b)	– 1.0	this is negative, so the sign bit is S = 1
	1.0 = 1.020
	1.M = 1.0	so M = 0000
	P = 0	so P + 127 = 127	= 0111 11112

Concatenate S | (P + 127) | M	1 0111 1111 0000
	Group by 4’s from the left	1011 1111 1000 0
	Pad out the last to four bits	1011 1111 1000 0000
	Convert to hex digits	BF80
	Pad out to eight hexadecimal digits		0xBF80 0000

c)	7.625	this is non-negative, so the sign bit is S = 0
	Convert 7.625 to binary.
		7 = 4 + 2 + 1		01112
		0.625 = 5/8 = 1/2 + 1/8	.1012
		7.625 		111.1012
	Normalize by moving the binary point
	two places to the left.		1.1110122
	Thus saying that 22  7.625 < 23.

1.M = 1.11101   so M = 11101
	P = 2	so P + 127 = 129	= 1000 0001

Concatenate S | (P + 127) | M	0 1000 0001 11101
	Group by 4’s from the left	0100 0000 1111 01
	Pad out the last to four bits	0100 0000 1111 0100
	Covert to hex digits	40F4		0x40F4 0000

d)	– 8.75	this is negative, so S = 1
	8.75 = 8 + 1/2 + 1/4	1000.11
			1.0001123

1.M = 1.00011  so M = 00011
	P = 3	so P + 127 = 130	1000 0010
	Concatenate S | (P + 127) | M	1 1000 0010 00011
	Group by 4’s from the left	1100 0001 0000 11
	Pad out the last to four bits	1100 0001 0000 1100
	Convert to hex digits	C10C		0xC10C 0000

8	Give the value of the real number (in standard decimal representation) represented by 
the following 32-bit words stored as an IEEE standard single precision.
	a)	4068 0000	b)	42E8 0000	c)	C2E8 0000

ANSWERS:

The first step in solving these problems is to convert the hexadecimal to binary.

a)	4068 0000 = 0100 0000 0110 1000 0000 0000 0000 0000
Regroup to get 0  1000 0000  1101 0000 etc.

Thus	s = 0 (not a negative number)
		p + 127 = 100000002 = 12810, so p = 1
and		m = 1101, so 1.m = 1.1101 and the number is 1.110121 = 11.1012.

But 11.1012 = 2 + 1 + 1/2 + 1/8 = 3 + 5/8 = 3.625.

b)	42E8 0000 = 0100 0010 1110 1000 0000 0000 0000 0000
Regroup to get 0  1000 0101  1101 0000 etc
Thus	s = 0 (not a negative number)
		p + 127 = 100001012 = 128 + 4 + 1 = 133, hence p = 6
and		m = 1101, so 1.m = 1.1101 and the number is 1.110126 = 11101002 
But 11101002 = 64 + 32 + 16 + 4 = 96 + 20 = 116 = 116.0

c)	C2E80 0000 = 1100 0010 1110 1000 0000 0000 0000 0000
Regroup to get 1 1000 0101 1101 0000 etc.
Thus	s = 1 (a negative number) and the rest is the same as b).  So – 116.0

9	Consider the string of digits “2108”.
	a)	Show the coding of this digit string in EBCDIC.
	b)	How many bytes does this encoding take?

ANSWER:	F2 F1 F0 F8.  Four bytes.

10	Consider the positive number 2108.
	a)	Show the representation of this number in packed decimal format.
	b)	How many bytes does this representation take?

ANSWER:	To get an odd number of decimal digits, this must be represented
		with a leading 0, as 02108.  02 10 8C.  Three bytes.

11	The following block of bytes contains EBCDIC characters.
	Give the English sentence represented.

E3 C8 C5 40 C5 D5 C4 4B

Answer:	E3 C8 C5 40 C5 D5 C4 4B
		T  H  E     E  N  D  .  “THE END.”

12	Perform the following sums assuming that each is a hexadecimal number.
			Show the results as 16–bit (four hexadecimal digit) results.

a)	123C + 888C

b)	123C + 99D

ANSWER:	a)	In hexadecimal	C + C = 18		(12 + 12 = 24 = 16 + 8).
						3 + 8 + 1 = C		(Decimal 12 is 0xC)
						2 + 8 + 0 = A		(Decimal 10 is 0xA)
						1 + 8 + 0 = 9.		9AC8

b)	In hexadecimal	C + D = 19		(12 + 13 = 25 = 16 + 9)
						3 + 9 + 1 = D		(Decimal 13 is 0xD)
						2 + 9 + 0 = B		(Decimal 11 is 0xB)
						1 + 0 + 0 = 1		1BD9

Each of the previous two problems uses numbers written as 123C and 888C.

The numbers in the problem on packed decimal are not the same as those in
the problem on hexadecimal.  They just look the same.

Interpreted as packed decimal:	123C is interpreted as the positive number 123, and 
						888C is interpreted as the positive number 888.

If we further specify that each of these is to be read as an integer value, then we have 
the two numbers +123 and +888.

Interpreted as hexadecimal, 123C is interpreted as the decimal number 
1163 + 2162 + 316 + 12 = 4096 + 512 + 48 + 12 = 4,668

Interpreted as hexadecimal, 888C is interpreted as the decimal number 
8163 + 8162 + 816 + 12 = 32768 + 2048 + 128 + 12 = 34,968

As for the number 1011, it translates to 0x3F3.

13	The two–byte entry shown below can be interpreted in a number of ways.

VALUE     DC X‘021D’

a)	What is its decimal value if it is interpreted as an unsigned binary integer?
	b)	What is its decimal value if it is interpreted as a packed decimal value?

ANSWER: 	As a binary integer, its value is 2162 + 116 + 13 = 512 + 16 + 13 = 541.

As a packed decimal, this has value – 21.

14	A given computer uses byte addressing with the little-endian structure.
	The following is a memory map, with all values expressed in hexadecimal.

| Address | 104 | 105 | 106 | 107 | 108 | 109 | 10A | 10B | 10C | 10D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | C2 | 3F | 84 | 00 | 00 | 00 | 9C | C1 | C8 | C0 |

What is the value (as a decimal real number; e.g. 203.75 ) of the floating point 
	number stored at address 108?  Assume IEEE–754 single precision format.

Answer:  The first thing to notice is that the memory is byte-addressable.  That means that 
each address takes holds one byte or eight bits.  The IEEE format for single precision 
numbers calls for 32 bits to be stored, so the number takes four bytes of memory.

In byte addressable systems, the 32-bit entry at address 108 is stored in the four bytes
with addresses 108, 109, 10A, and 10B.  The contents of these are 00, 00, 9C, and C1.

The next thing to do is to get the four bytes of the 32-bit number in order.

This is a little-endian memory organization, which means that the LSB is stored at address 
108 and the MSB is stored as address 10B.  In correct order, the four bytes are
		C1  9C  00  00.  In binary, this becomes
		1100 0001   1001 1100   0000 0000   0000 0000.  Breaking into fields we get
		1100 0001 1001 1100 0000 0000 0000 0000, with the exponent field in bold, or

1  1000 0011  0011 1000.

Thus	s = 1		a negative number

e + 127 = 1000 00112 = 128 + 2 + 1 = 131, so e = 4
			m = 00111

So the number’s magnitude is 1.001112  24 = 10011.12 = 16 + 2 + 1 + 0.5 = 19.5,

and the answer is – 19.5.

15	Consider the 32-bit number represented by the eight hexadecimal digits
	BEEB 0000.  What is the value of the floating point number represented by this
	bit pattern assuming that the IEEE-754 single-precision standard is used?

ANSWER:  First recall the binary equivalents: B = 1011, E = 1110, and 0 = 0000.

Convert the hexadecimal string to binary

Hexadecimal:  B    E    E    B    0    0    0    0

Binary:     1011 1110 1110 1011 0000 0000 0000 0000

Regroup the binary according to the 1 | 8 | 23 split required by the format.

Binary:     1011 1110 1110 1011 0000 0000 0000 0000
Split:      1 011 1110 1 110 1011 0000 0000 0000 0000
Regrouped:  1  0111 1101  1101 0110 0000 0000 0000 000

The fields in the expression are now analyzed.

Sign bit:	S = 1		this will become a negative number

Exponent:

The field contains 0111 1101, or 64 + 32 + 16 + 8 + 4 + 1 = 96 + 24 + 5 = 125.  This 
number may be more easily derived by noting that 0111 1111 = 127 and this is 2 less.

The exponent is given by P + 127 = 125, or P = – 2.  The absolute value of the number 
being represented should be in the range [0.25, 0.50), or 0.25  N  < 0.50.

Mantissa:

The mantissa field is 1101 0110, so 1.M = 1.1101 0110.

In decimal, this equals 1 + 1/2 + 1/4 + 1/16 + 1/64 + 1/128, also written as
				(128 + 64 + 32 + 8 + 2 + 1) / 128 = (192 + 40 + 3) / 128 = 235 / 128.

The magnitude of the number equals 235 / 128  1/4 = 235 / 512 = 0.458984375.

16	The following are two examples of the hexadecimal representation of 
		floating–point numbers stored in the IBM single–precision format.
		Give the decimal representation of each.  Fractions (e.g., 1/8) are acceptable.

a)	C1 64 00 00

b)	3F 50 00 00

ANSWER:	a)	First look at the sign and exponent byte.  This is 0xC1, or 1100 0001.

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |

The sign bit is 1, so this is a negative number.
Stripping the sign bit, the exponent field is 0100 0001 or 0x41 = decimal 65.
We have (Exponent + 64) = 65, so the exponent is 1.

The value is 161F, where F is 0x64, or 6/16 + 4/256.
The magnitude of the number is 161(6/16 + 4/256) = 6 + 4/16 = 6.25.  The value is –6.25.

b)	First look at the sign and exponent byte.  This 0x3F, or 0011 1111

The sign bit is 0, so this is a non–negative number.
The exponent field is 0x3F = 316 + 15 = decimal 63 = 64 – 1.

The value is 16–1F, where F is 0x50, or 5/16.
The magnitude of the number is 16–1(5/16) = 5/256 = 0.01953125.

17	These questions refer to the IBM Packed Decimal Format.

a)	How many bytes are required to represent a 3–digit integer?

b)	Give the Packed Decimal representation of the positive integer 123.

c)	Give the Packed Decimal representation of the negative integer –107.

ANSWER:	Recall that each decimal digit is stored as a hexadecimal digit, and 
			that the form calls for one hexadecimal digit to represent the sign.

a)	One needs four hexadecimal digits, or two bytes, to represent three decimal digits.

b)		12 3C		c)	10 7D

18	These questions also refer to the IBM Packed Decimal Format.

a)	How many decimal digits can be represented in Packed Decimal form 
		if three bytes (8 bits each) are allocated to store the number?

b)	What is the Packed Decimal representation of the largest integer stored in 3 bytes?

ANSWER:	Recall that N bytes will store 2N hexadecimal digits.  With one of these 
			reserved for the sign, this is (2N – 1) decimal digits.

a)	3 bytes can store five decimal digits.

b)	The largest integer is 99,999.  It is represented as 99 99 9C.

19	Convert the following numbers to their representation IBM Single Precision
	floating point and give the answers as hexadecimal digits.

a)	123.75

b)	–123.75

ANSWER:
	a)	First convert the number to hexadecimal.

The whole number conversion:	123 / 16 = 7 with remainder = 11 (B)
						7 / 16 = 0 with remainder 7.   123 = 7B.

The fractional part conversion:	.7516 = 12 (C).  The number is 7B.C

The number can be represented as 162  0.7BC; the exponent is 2.

The exponent stored with excess 64, thus it is 66 or X‘42’.

Appending the fractional part, we get X‘427BC’.

Add three hexadecimal zeroes to pad out the answer to X‘427B C000’

b)	The only change here is to add the sign bit as the leftmost bit.

In the positive number, the leftmost byte was X‘42’, which in binary would be

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |

Just flip the bit in position 0 to get the answer for the leftmost byte.

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |

This is X‘C2’.  The answer to this part is X‘C27B C000’

Convert the following numbers to their representation in packed decimal.
	Give the hexadecimal representation with the proper number of hexadecimal digits.

a)	123.75

b)	–123.7

ANSWER:	a)	12375 has five digits.  It is represented as 12 37 5C.

b)	1237 has four digits.  Expand to 01237 and represent as 01 23 7D.

20	Give the correct Packed Decimal representation of the following numbers.

a)	31.41	b)	–102.345	c)	1.02345

ANSWER:	Recall that the decimal is not stored, and that we need to have an odd 
			count of decimal digits.

a)	This becomes	3141,	or 03141.	03141C

b)	This becomes	102345,	or 0102345	0102345D

c)	This also becomes	102345,	or 0102345	0102345C.

21	Perform the following sums of numbers in Packed Decimal format. Convert to 
		standard integer and show your math.  Use Packed Decimal for the answers.

a)	025C + 085C			d)	666D + 444D

b)	032C + 027D			e)	091D + 0C

c)	10003C + 09999D

ANSWER:	Just do the math required and convert back to standard 
Packed Decimal format.

a)	025C + 085C represents 25 +85 = 110.  This is represented as 110C.

b)	032C + 027D represents 32 –27 = 5.  This is represented as 5C.

c)	10003C + 09999D represents 10003 –9999 = 4.  This is represented as 4C.

d)	666D + 444D represents –666 –444 = –1110.  This is represented as 01 11 0D.

e)	091D + 0C represents –91 +0 = –91  This is represented as 091D.

22	These questions concern 10–bit integers, which are not common.

a)	What is the range of integers storable in 10–bit unsigned binary form?

b)	What is the range of integers storable in 10–bit two’s–complement form?

c)	Represent the positive number 366 in 10–bit two’s–complement binary form.

d)	Represent the negative number –172 in 10–bit two’s–complement binary form.

e)	Represent the number 0 in 10–bit two’s–complement binary form.

ANSWER:	Recall that an N–bit scheme can store 2N distinct representations.
			For unsigned integers, this is the set of integers from 0 through 2N – 1.
			For 2’s–complement, this is the set from – (2N–1) through 2N–1 – 1.

a)	For 10–bit unsigned the range is 0 though 210 – 1, or 0 through 1023.

b)	For 10–bit 2’s–complement, this is – (29) through 29 – 1, or – 512 through 511.

c)			366 / 2	= 183	remainder = 0
				183 / 2	= 91	remainder = 1
				91 / 2	= 45	remainder = 1
				45 / 2	= 22	remainder = 1
				22 / 2	= 11	remainder = 0
				11 / 2	= 5	remainder = 1
				5 / 2	= 2	remainder = 1
				2 / 2	= 1	remainder = 0
				1 / 2	= 0	remainder = 1.	READ BOTTOM TO TOP!

The answer is 1 0110 1110, or 01 0110 1110, which equals 0x16E.
0x16E = 1256 + 616 + 14 = 256 + 96 + 14 = 256 + 110 = 366.

The number is not negative, so we stop here.  Do not take the two’s complement unless the
number is negative.

d)			172 / 2	= 86	remainder = 0
				86 / 2	= 43	remainder = 0
				43 / 2	= 21	remainder = 1
				21 / 2	= 10	remainder = 1
				10 / 2	= 5	remainder = 0
				5 / 2	= 2	remainder = 1
				2 / 2	= 1	remainder = 0
				1 / 2	= 0	remainder = 1.	READ BOTTOM TO TOP!

This number is 1010 1100, or 00 1010 1100, which equals 0x0AC.
0xAC = 1016 + 12 = 160 + 12 = 172.

The absolute value:	00 1010 1100
Take the one’s complement:	11 0101 0011
Add one:			           1
The answer is:	11 0101 0100	or 0x354.

e)	The answer is	00 0000 0000.
		You should just know this one.

23	These questions IBM Packed Decimal Form.

a)	Represent the positive number 366 as a packed decimal with fewest digits.

d)	Represent the negative number –172 as a packed decimal with fewest digits.

e)	Represent the number 0 as a packed decimal with fewest digits.

ANSWER:	a)	366C
			b)	172D
			c)	0C	(not 0D, which is incorrect)