Advanced Features of Macro Instructions

This lecture will focus on some of the advanced features of the 
macro language as implemented by the IBM/System 360 assembler.

We shall focus on our stack handling macros.

Some of the features to be covered by this lecture include.

1.	The use of concatenation to generate type–specific instructions.

2.	Some standard system variable symbols.

3.	The use of one system variable symbol to solve the branch problem.

4.	Conditional assembly.

5.	The use of conditional assembly as a help in writing STKPOP.

7.	The ABEND macro and its use in signaling run–time errors.

8.	A completed version of our stack macros.

Concatenation: Building Operations

In a model statement, it is possible to concatenate two strings of characters.

Consider the macro prototype to load a register from one of several sources.
Note the use of the string “&NAME” to allow this to be a branch target.

MACRO

&NAME    LOAD &REG,&TYPE,&ARG

&NAME    L&TYPE &REG,&ARG

MEND

Consider a number of invocations.

LOAD R7,R,R6	becomes		LR R7,R6

LOAD R7,H,HW	becomes		LH R7,HW

LOAD R7,,FW	becomes		L R7,FW

Note here: the second argument is empty.  The empty string is concatenated to “F”.

We shall now extend the stack operations to push and pop contents of 
half–words and full–words, as well as registers.

Pushing from Various Sources

We look first at the handling of our STKPUSH.  The only restriction on the 
stack is that every value pushed be treated as a 32–bit fullword.

As a result, a 16–bit halfword will be sign–extended to a 32–bit fullword 
before being pushed onto the stack.  This is similar to the function of the 
LH instruction, which loads a register from a halfword.

The key instruction in the original STKPUSH macro is the following.

ST  &R,0(3,2)     STORE THE ITEM INTO THE STACK

In this case, the item to be placed on the stack is found in the register 
indicated by the symbolic parameter &R.

The way to extend this instruction to all data types is as follows.

1.	Select a register to be a fixed source for the word on the stack, and

2.	Construct instructions to load that fixed register from the source.

What Shall Be Stored on the Stack?

At this point, we have a decision to make.  What data types to store?

The size restriction on the stack limits the simple choices to addresses and the 
contents of registers, halfwords, and fullwords.

We must select a working register for the new macro.  I select R4.
The “key code” becomes as follows.

Stacking an address	LA R4,&ARG      Load address into R4.
						ST R4,&R,0(3,2)

Stacking a halfword	LH R4,&ARG     Load halfword into R4.
						ST R4,&R,0(3,2)

Stacking a fullword	L  R4,&ARG     Load fullword into R4.
						ST R4,&R,0(3,2)

Stacking a register	LR R4,&ARG     Load value from source
                              register into R4.
						ST R4,&R,0(3,2)

Passing the Type in a Macro Invocation

The solution adopted to the problem above is to pass the type in the 
macro call and use concatenation to build the load operator.

Here is some code taken from a macro definition that has been run and tested.  
First, we show the macro prototype.

&L2      STKPUSH &ARG,&TYP

Next we show the “key instruction” in the macro body.

L&TYP R4,&ARG

Here are four typical invocations of the macro.

STKPUSH R7,R      PUSH VALUE IN REGISTER.

STKPUSH HHW,H     PUSH A HALFWORD VALUE.

STKPUSH FFW,A     PUSH AN ADDRESS.

STKPUSH FFW       PUSH A FULLWORD.

Note that the last invocation lacks a second argument.  In the expansion, this 
causes &TYP to be set to ‘ ’, a blank; “L&TYP” becomes “L ”.

The Macro Definition

Here is the definition for the macro at this stage of its development.

MACRO

&L2      STKPUSH &ARG,&TYP

&L2      LH    R3,STKCOUNT

SLA   R3,2

LA    R2,THESTACK

L&TYP R4,&ARG

ST    R4,0(3,2)

LH    R3,STKCOUNT

AH    R3,=H'1'

STH   3,STKCOUNT

MEND

Again, the “&L2” allows the macro invocation to be a branch target.

Some Invocations of this Macro

91          STKPUSH R7,R

92+         LH    R3,STKCOUNT

93+         SLA   R3,2

94+         LA    R2,THESTACK

95+         LR    R4,R7

96+         ST    R4,0(3,2)

97+         LH    R3,STKCOUNT

98+         AH    R3,=H'1'

99+         STH   3,STKCOUNT

100          STKPUSH HHW,H

101+         LH    R3,STKCOUNT

102+         SLA   R3,2

103+         LA    R2,THESTACK

104+         LH    R4,HHW

105+         ST    R4,0(3,2)

106+         LH    R3,STKCOUNT

107+         AH    R3,=H'1'

108+         STH   3,STKCOUNT

More Invocations of this Macro

109          STKPUSH FFW

110+         LH    R3,STKCOUNT

111+         SLA   R3,2

112+         LA    R2,THESTACK

113+         L     R4,FFW

114+         ST    R4,0(3,2)

115+         LH    R3,STKCOUNT

116+         AH    R3,=H'1'

117+         STH   3,STKCOUNT

118          STKPUSH FFW,A

119+         LH    R3,STKCOUNT

120+         SLA   R3,2

121+         LA    R2,THESTACK

122+         LA    R4,FFW

123+         ST    R4,0(3,2)

124+         LH    R3,STKCOUNT

125+         AH    R3,=H'1'

126+         STH   3,STKCOUNT

NOTE: The originals of the program listing are found at the end of the slides.

Saving the Work Registers

As written, this macro has the side effect of changing the values of three 
registers: R2, R3, & R4.  The value of R4 is preserved only if it is being pushed.

We should write macros so that they operate without side effects.  The only 
way to do this is to save and restore the values of the work registers.

There are many ways to do this.  The simplest is to alter the stack data structure.
Here is the new version.

STKCOUNT DC H‘0’    NUMBER OF ITEMS STORED ON STACK

STKSIZE  DC H‘64’   MAXIMUM STACK CAPACITY

STKSAV2  DC F‘0’    SAVES CONTENTS OF R2

STKSAV3  DC F‘0’    SAVES CONTENTS OF R3

STKSAV4  DC F‘0’    SAVES CONTENTS OF R4

THESTACK DC 64F‘0’  THE STACK HOLDS 64 FULLWORDS

This new definition does not alter the STKINIT macro.  It does affect the 
other two macros: STKPOP and STKPUSH.  We illustrate the latter.

The First Revision of STKPUSH

Here is the revision that allows the work registers to be saved.

MACRO

&L2      STKPUSH &ARG,&TYP

&L2      ST    R2,STKSAV2   THE ORDER OF SAVING

ST    R3,STKSAV3   IS NOT IMPORTANT.

ST    R4,STKSAV4

LH    R3,STKCOUNT

SLA   R3,2

LA    R2,THESTACK

L&TYP R4,&ARG

ST    R4,0(3,2)

LH    R3,STKCOUNT

AH    R3,=H'1'

STH   R3,STKCOUNT

L     R4,STKSAV4   THE ORDER OF RESTORATION

L     R3,STKSAV3   IS NOT IMPORTANT EITHER.

L     R2,STKSAV2

MEND

The Status of the Macros at This Point

There are a few issues to be addressed at this point.

The only macro that will not change is the initialization macro, STKINIT.

1.	We have not yet dealt with generalizing the STKPOP macro.

2.	We have not yet dealt with either the stack empty problem or 
		that of the stack being full.  Each has to be addressed.

Each of these issues demands the use of techniques we have not yet discussed.

Consider the first problem.  We shall want to pop the following from the stack: 
register values, halfwords, and fullwords.  The type for the argument refers to 
the destination; an address can be popped into either a register or fullword.

In order to see the problem for STKPOP, consider the “key instruction”.

Halfword:	STH R4,&ARG

Fullword:	ST R4,&ARG

Register:	LR &ARG,R4  No STR for store register.

We could write a STR macro, but I want to use another solution.

Some System Variable Symbols

There are a number of system variable symbols.  I mention three.

&SYSDATE		The system date, in the 8 character form “MM/DD/YY”.
					Use in the form of a declaration of initialized storage, as in
					TODAY    DC C‘&SYSDATE’

&SYSTIME		The system time of day, in the five character form “HH.MM”.
					Also used in the form of a declaration, as in
					NOW      DC C‘&SYSTIME’

&SYSNDX		The macro expansion index.  For the first macro expansion, 
					the Assembler initializes &SYSNDX to the string “0001”.
					Each macro invocation increases the value represented by 1, 
					giving rise to the sequence “0001”, “0002”, “0003”, etc.

The &SYSNDX system variable symbol can prevent a macro from generating 
duplicate labels.  The system symbol is concatenated to a leading character, 
which begins the label and must be unique within the macro definition.

More on the Macro Expansion Index

First consider the following string, used as a label in a macro definition.

L&SYSNDX L R4,STKSAV4

Note that the string “L&SYSNDX”, as written, contains eight characters: 
the initial character “L” followed by the 7 character sequence “&SYSNDX”.

On expansion, this will be converted to labels such as “L0001”, “L0002”, etc.

In the macro definition, this takes the maximum eight characters allowed for 
a properly formatted listing.  For this reason, I suggest that the better form for 
the label in the macro definition is Single_Letter&SYSNDX.

In actual fact, the requirement for the leading characters, to which the &SYSNDX 
is to be appended can be any sequence of one to four characters, provided only 
that the first character is a letter.  Thus the following are valid.

A12&SYSNDX ...    This label might become A120003.

WXYZ&SYSNDX ...   This might become WXYZ0117.

I suggest use of a single leading letter, this allows 26 labels per macro.

A Simple Example of Label Generation

Consider the simple macro used for packed division in the previous lecture.
We adapt it to prevent division by zero.

MACRO

&LABEL   DIVID &QUOT,&DIVIDEND,&DIVISOR

&LABEL   ZAP   &QOUT,&DIVIDEND

CP    &DIVISOR,=P‘0’  IS IT ZERO

BNE   A&SYSNDX        NO, DIVISION IS OK

ZAP   &QUOT,=P‘0’     YES, SET QUOTIENT TO 0

B     B&SYSNDX

A&SYSNDX DP    &QUOT,&DIVISOR

B&SYSNDX NOPR  R3              DO NOTHING

MEND

Note that the format of the NOPR instruction requires a register number 
(here R3), even though the instruction does nothing.

Sample Expansion of the Macro

With the above definition, consider the following expansions.

A10START DIVID X,Y,Z

+A10START ZAP   X,Y

+         CP    Z,=P‘0’     IS IT ZERO

+         BNE   A0001       NO, DIVISION IS OK

+         ZAP   X,=P‘0’     YES, SET QUOTIENT TO 0

+         B     B0001

+A0001    DP    X,Z

+B0001    NOPR  R3          DO NOTHING

A20DOIT  DIVID A,B,C

+A20DOIT  ZAP   A,B

+         CP    C,=P‘0’     IS IT ZERO

+         BNE   A0002       NO, DIVISION IS OK

+         ZAP   X,=P‘0’     YES, SET QUOTIENT TO 0

+         B     B0002

+A0002    DP    A,C

+B0002    NOPR  R3          DO NOTHING

Note that each invocation has distinct labels.  This removes the name clashes.

Another Design Strategy for DIVID

In this variant, a zero divisor will cause the program to terminate abnormally.

MACRO

&LABEL   DIVID &QUOT,&DIVIDEND,&DIVISOR

&LABEL   ZAP   &QOUT,&DIVIDEND

CP    &DIVISOR,=P‘0’  IS IT ZERO

BNE   A&SYSNDX        NO, DIVISION IS OK

ABEND                 INVOKE THE MACRO TO
                               TERMINATE EXECUTION.

A&SYSNDX DP    &QUOT,&DIVISOR

MEND

The First Revision of STKINIT

Here is a revision of the STKINIT code that allows initialization of its size.

35          MACRO

36 &L1      STKINIT &SIZE

37 &L1      ST R3,STKSAV3

38          SR R3,R3

39          STH R3,STKCOUNT

40          L  R3,STKSAV3

41          B  L&SYSNDX

42 STKCOUNT DC H'0'

43 STKSIZE  DC H'&SIZE'

44 STKSAV2  DC F'0'

45 STKSAV3  DC F'0'

46 STKSAV4  DC F'0'

47 THESTACK DC &SIZE.F'0'

48 L&SYSNDX SLA R3,0

49          MEND

Note the “.” in the definition of THESTACK.  This concatenates the value 
of the symbolic parameter with “F‘0’”, as in “128F‘0’”

The Second Revision of STKPUSH

MACRO

&L2      STKPUSH &ARG,&TYP

&L2      ST    R3,STKSAV3

LH    R3,STKCOUNT  GET COUNT OF ITEMS ON THE STACK

CH    R3,STKSIZE   IS THE STACK FULL?

BNL   Z&SYSNDX     YES, DO NOT ADD ANOTHER.

ST    R4,STKSAV4   NO, WE CAN PUSH ANOTHER ITEM.

ST    R2,STKSAV2   START BY SAVING THE OTHER 2 REGISTERS

SLA   R3,2         MULTIPLY THE INDEX BY 4.

LA    R2,THESTACK

L&TYP R4,&ARG      FORM THE ADDRESS

ST    R4,0(3,2)    STORE THE ITEM

LH    R3,STKCOUNT  GET THE OLD COUNT OF ITEMS

AH    R3,=H'1'     INCREMENT THE COUNT BY 1

STH   R3,STKCOUNT  STORE THE CURRENT COUNT

L     R4,STKSAV4   RESTORE THE REGISTERS.

L     R2,STKSAV2

Z&SYSNDX L     R3,STKSAV3

MEND

This is the final version of the STKPUSH macro.

We must discuss another basic topic before addressing STKPOP.

Conditional Assembly

We have already seen how concatenation can be used to construct 
different instructions in a macro expansion.

We now investigate conditional assembly, in which the expansion of a macro 
can lead to a number of distinct code sequences.

Conditional assembly permits the testing of attributes such as data format, data 
value, or field length, and to use the results of such testing to generate source 
code that is specific to the case in question.

This lecture will focus on five specific conditional assembly instructions.

AGO		an unconditional branch

AIF		a conditional branch.  This means “Ask If”.

ANOP		A NOP that can be the branch target for either AGO or AIF.

MNOTE	print a programmer defined message at assembly time

MEXIT		exit the macro definition.

Attributes for Use by Conditional Assembly

The assembler can generate code specified by certain attributes of 
the arguments to the macro definition at the time it is expanded.

There are six types of attributes that can be associated with a parameter.
Here are three if the more useful attributes.

L’		Length		The length of the symbolic parameter

I’		Integer		The integer attribute of a fixed–point, 
						floating–point, or packed decimal number.

T’		Type		The type of the parameter, as specified by the
						DC or DS declaration with which it is defined.

Some types for the T’ attribute are as follows.

A	Address	C	Character	H	Halfword	P	Packed Decimal

B	Binary	F	Fullword	I	Instruction	X	Hexadecimal

The Ask If (AIF) Instruction

The AIF instruction has two parts.

1.	A logical expression in parentheses, and

2.	A sequence symbol immediately following, which serves 
		as the branch target.

The AIF logical expression may use the following relational operators, which 
are quite similar to those seen in early versions of the FORTRAN language.

EQ	Equal To	NE	Not Equal To

LT	Less Than	GE	Greater Than or Equal To

GT	Greater Than	LE	Less Than or Equal To

If the type of &AMT is packed, go to .B23PACK

AIF(T’&AMT EQ ‘P’).B23PACK

If the type of &LINK is not an instruction, go to .R30ERROR

AIF(T’&LINK NE ‘I’).R30ERROR

Testing the Value of a Symbolic Parameter

What we want for the STKPOP instruction is a conditional assembly 
based on the value of the second parameter.

The prototype will be something like
&L1      STKPOP &ARG,&TYP

What we want to issue is an AIF statement such as
AIF (&TYP EQ ‘R’).ISREG

There is a well–known peculiarity in assembler language, not just in the 
IBM Assembler, that disallows this straightforward construct.

We must put the symbolic parameter in single quotes.  The statement is thus:

AIF (‘&TYP’ EQ ‘R’).ISREG

If &TYP is the character R, the logical expression becomes (‘R’ EQ ‘R’),
which immediately evaluates to True, and the branch is taken.

Reference
Page 384,	High Level Assembler for z/OS & z/VM & z/VSE Language 
			Reference Manual, Release 6 (July 2008), SC26–4940–05

Targets for Use by Conditional Assembly

Each of the AGO and AIF instructions is a branch instruction that takes 
effect at assembly time.  Neither persists into the assembly source code.

It should be expected that the targets for either of these conditional 
assembly branch instructions should be of a distinct type.

The targets for these are called sequence symbols.

The format of a sequence symbol is as follows.
A sequence symbol begins with a period (.) followed by one to seven letters 
or digits, the first of which must be a letter.

Unlike the symbols created by use of the &SYSNDX system symbol, 
sequence symbols do not persist into assembly time, and thus 
cannot generate a name conflict for the assembler.

A Sample of Conditional Assembly

Here is the DIVID macro, with conditional assembly instructions to 
insure that it is expanded only for parameters that are packed decimal.

MACRO

&LABEL   DIVID &QUOT,&DIVIDEND,&DIVISOR

AIF   (T’&QUOT NE ‘P’).NOTPACK

AIF   (T’&DIVIDEND NE T’&QUOT).NOTPACK

AIF   (T’&DIVISOR NE T’&QUOT).NOTPACK

AGO   .DOIT

.NOTPAK  MNOTE  ‘ONE PARAMETER IS NOT PACKED DECIMAL’

MEXIT

.DOIT    ANOP

&LABEL   ZAP   &QOUT,&DIVIDEND

CP    &DIVISOR,=P‘0’  IS IT ZERO

BNE   A&SYSNDX        NO, DIVISION IS OK

ZAP   &QUOT,=P‘0’     YES, SET QUOTIENT TO 0

B     B&SYSNDX

A&SYSNDX DP    &QUOT,&DIVISOR

B&SYSNDX NOPR  R3              DO NOTHING

MEND

Some Examples of the Conditional Assembly Divide Macro

In the following, assume that each of X, Y, and Z is defined by a DC 
statement as packed decimal, but that A, B, and C are defined as halfwords.

Here are some possible expansions.

F10DOIT  DIVID X,Y,Z

+F10DOIT  ZAP   X,Y

+         CP    Z,=P‘0’         IS IT ZERO

+         BNE   A0032           NO, DIVISION IS OK

+         ZAP   X,=P‘0’         YES, SET QUOTIENT TO 0

+         B     B0032

+A0032    DP    X,Z

+B0032    NOPR  R3              DO NOTHING

F25NODO  DIVID A,B,C

+ONE PARAMETER IS NOT PACKED DECIMAL

The Original Definition of Macro STKPOP

We now begin our redefinition of the STKPOP macro.
We begin with the original definition, which popped a value into a register.

*STKPOP

MACRO

&L3      STKPOP &R

&L3      LH  3,STKCOUNT   GET THE STACK COUNT

SH  3,=H’1’      SUBTRACT 1 WORD OFFSET OF TOP

STH 3,STKCOUNT   STORE AS NEW SIZE

SLA 3,2          BYTE OFFSET OF STACK TOP

LA  2,THESTACK   ADDRESS OF STACK BASE

L   &R,0(3,2)    LOAD ITEM INTO THE REGISTER.

MEND

*

Again, this macro has one symbolic parameter: &R.  Again, a register number.

We want to expand this definition in a number of ways.

We begin by introducing the type &TYP.

At this point, it will become necessary to have another work register.

Mechanics of the Revised STKPOP

The new design will use register R4 to transfer the value at the top of the stack.

The new prototype will be as follows.

&L3      STKPOP &ARG,&TYP

Each type of instruction will include the following as the first statement 
in the “key code” – that which actually places the value into the destination.

L   R4,0(3,2)    LOAD ITEM INTO REGISTER R4.

The second statement of the “key code” depends on the type of the destination.

&TYP == H        STH R4,&ARG

&TYP == F        ST  R4,&ARG

&TYP == A        ST  R4,&ARG  (SAME AS FULLWORD)

&TYP == R        LR &ARG,R4   COPY R4 INTO REGISTER

Again, I could define a STR macro and avoid the use of conditional assembly.
For a number of reasons, I have chosen not to do so.

The Key Code as Reflected in Conditional Assembly

Again, the new prototype will be as follows.

&L3      STKPOP &ARG,&TYP

Here is the key code section, with the conditional assembly.

The first statement is common to all types.

L   R4,0(3,2)    LOAD ITEM INTO REGISTER R4.

AIF (‘&TYPE’ EQ ‘R’).ISREG

ST&TYP R4,&ARG

AGO .CONT

.ISREG   LR &ARG,R4

.CONT    The next statement.

STKPOP: Revision 2

Here I am going to add some code to save and restore the work registers.

MACRO

&L3      STKPOP &ARG,&TYP

&L3      ST  R2,STKSAV2

ST  R3,STKSAV3

ST  R4,STKSAV4

LH  R3,STKCOUNT   GET THE STACK COUNT

SH  R3,=H’1’      SUBTRACT 1 WORD OFFSET OF TOP

STH R3,STKCOUNT   STORE AS NEW SIZE

SLA R3,2          BYTE OFFSET OF STACK TOP

LA  R2,THESTACK   ADDRESS OF STACK BASE

L   R4,0(3,2)    LOAD ITEM INTO REGISTER R4.

AIF (‘&TYPE’ EQ ‘R’).ISREG

ST&TYP R4,&ARG

AGO .CONT

.ISREG   LR &ARG,R4

.CONT    L   R4,STKSAV4

L   R3,STKSAV3

L   R2,STKSAV2

MEND

STKPOP: The Complete Version

MACRO

&L3      STKPOP &ARG,&TYP

&L3      ST  R3,STKSAV3

LH  R3,STKCOUNT   GET THE STACK COUNT

CH  R3,=H‘0’      IS THE COUNT POSITIVE

BNH Z&SYSNDX      NO, WE CANNOT POP.

SH  R3,=H’1’      SUBTRACT 1 WORD OFFSET OF TOP

STH R3,STKCOUNT   STORE AS NEW SIZE

SLA R3,2          BYTE OFFSET OF STACK TOP

ST  R2,STKSAV2    SAVE REGISTER R2

ST  R4,STKSAV4    SAVE REGISTER R4

LA  R2,THESTACK   ADDRESS OF STACK BASE

L   R4,0(3,2)    LOAD ITEM INTO REGISTER R4.

AIF (‘&TYPE’ EQ ‘R’).ISREG

ST&TYP R4,&ARG

AGO .CONT

.ISREG   LR &ARG,R4

.CONT    L   R4,STKSAV4

L   R2,STKSAV2

Z&SYSNDX L   R3,STKSAV3

MEND

Original Code for the Macro Expansions

33 *        MACRO DEFINITIONS

34 *

35          MACRO

36 &L2      STKPUSH &ARG,&TYP

37 &L2      LH    R3,STKCOUNT

38          SLA   R3,2

39          LA    R2,THESTACK

40          L&TYP R4,&ARG

41          ST    R4,0(3,2)

42          LH    R3,STKCOUNT

43          AH    R3,=H'1'

44          STH   3,STKCOUNT

45          MEND

46 *

89 *        SOME MACRO INVOCATIONS

90 *

91          STKPUSH R7,R

00004A 4830 C0C6            000CC    92+         LH    R3,STKCOUNT

00004E 8B30 0002            00002    93+         SLA   R3,2

000052 4120 C0CA            000D0    94+         LA    R2,THESTACK

000056 1847                          95+         LR    R4,R7

000058 5043 2000            00000    96+         ST    R4,0(3,2)

00005C 4830 C0C6            000CC    97+         LH    R3,STKCOUNT

000060 4A30 C43A            00440    98+         AH    R3,=H'1'

000064 4030 C0C6            000CC    99+         STH   3,STKCOUNT

100          STKPUSH HHW,H

000068 4830 C0C6            000CC   101+         LH    R3,STKCOUNT

00006C 8B30 0002            00002   102+         SLA   R3,2

000070 4120 C0CA            000D0   103+         LA    R2,THESTACK

000074 4840 C1CE            001D4   104+         LH    R4,HHW

000078 5043 2000            00000   105+         ST    R4,0(3,2)

00007C 4830 C0C6            000CC   106+         LH    R3,STKCOUNT

000080 4A30 C43A            00440   107+         AH    R3,=H'1'

000084 4030 C0C6            000CC   108+         STH   3,STKCOUNT

109          STKPUSH FFW

000088 4830 C0C6            000CC   110+         LH    R3,STKCOUNT

00008C 8B30 0002            00002   111+         SLA   R3,2

000090 4120 C0CA            000D0   112+         LA    R2,THESTACK

000094 5840 C1CA            001D0   113+         L     R4,FFW

000098 5043 2000            00000   114+         ST    R4,0(3,2)

00009C 4830 C0C6            000CC   115+         LH    R3,STKCOUNT

0000A0 4A30 C43A            00440   116+         AH    R3,=H'1'

0000A4 4030 C0C6            000CC   117+         STH   3,STKCOUNT

118          STKPUSH FFW,A

0000A8 4830 C0E6            000EC   119+         LH    R3,STKCOUNT

0000AC 8B30 0002            00002   120+         SLA   R3,2

0000B0 4120 C0EA            000F0   121+         LA    R2,THESTACK

0000B4 4140 C1EA            001F0   122+         LA    R4,FFW

0000B8 5043 2000            00000   123+         ST    R4,0(3,2)

0000BC 4830 C0E6            000EC   124+         LH    R3,STKCOUNT

0000C0 4A30 C45A            00460   125+         AH    R3,=H'1'

0000C4 4030 C0E6            000EC   126+         STH   3,STKCOUNT

127 *

136 **********************************

Revised Code for the Macros

The next few pages show the listing of the final forms of the macros, as actually 
coded and tested.  These are followed by listings of the expanded macros.

002900 *

002910          MACRO

002911 &L1      STKINIT

002912 &L1      ST R3,STKSAV3

002913          SR R3,R3

002914          STH R3,STKCOUNT           CLEAR THE COUNT

002915          L  R3,STKSAV3

002920          MEND

002930 *

003000          MACRO

003100 &L2      STKPUSH &ARG,&TYP

003110 &L2      ST    R3,STKSAV3         SAVE REGISTER R3

003200          LH    R3,STKCOUNT        GET THE CURRENT SIZE

003210          CH    R3,STKSIZE         IS THE STACK FULL?

003220          BNL   Z&SYSNDX           YES, DO NOT PUSH

003230          ST    R4,STKSAV4         OK, SAVE R2 AND R4

003240          ST    R2,STKSAV2

003300          SLA   R3,2               MULTIPLY BY FOUR

003310          LA    R2,THESTACK        ADDRESS OF STACK START

003320          L&TYP R4,&ARG            LOAD R4 WITH VALUE

003330          ST    R4,0(3,2)          STORE INTO THE STACK

003331          LH    R3,STKCOUNT

003332          AH    R3,=H'1'

003333          STH   3,STKCOUNT

003334          L     R4,STKSAV4

003335          L     R2,STKSAV2

003336 Z&SYSNDX L     R3,STKSAV3

003337          MEND

003338 *

003339 *

003340          MACRO

003341 &L3      STKPOP &ARG,&TYP

003342 &L3      ST    R3,STKSAV3

003343          LH    R3,STKCOUNT     GET THE STACK COUNT

003344          CH    R3,=H'0'        IS THE COUNT POSITIVE?

003345          BNH   Z&SYSNDX        NO, WE CANNOT POP

003346          SH    R3,=H'1'        SUBTRACT 1 WORD OFFSET

003347          STH   R3,STKCOUNT     STORE THE NEW SIZE

003348          SLA   R3,2            BYTE OFFSET OF STACK TOP

003349          ST    R2,STKSAV2      SAVE REGISTER R2

003350          ST    R4,STKSAV4      SAVE REGISTER R4

003351          LA    R2,THESTACK     ADDRESS OF STACK BASE

003352          L     R4,0(3,2)       LOAD ITEM INTO R4

003353          AIF   ('&TYP' EQ 'R').ISREG

003354          ST&TYP R4,&ARG

003355          AGO .CONT

003356 .ISREG   LR &ARG,R4

003357 .CONT    L   R4,STKSAV4

003358          L   R2,STKSAV2

003359 Z&SYSNDX L   R3,STKSAV3

003360          MEND

003361 *

Revised Code for the Macro Expansions

128 *        SOME MACRO INVOCATIONS

129 *

130          STKINIT

00004A 5030 C22E            00234   131+         ST R3,STKSAV3

00004E 1B33                         132+         SR R3,R3

000050 4030 C226            0022C   133+         STH R3,STKCOUNT

000054 5830 C22E            00234   134+         L  R3,STKSAV3

135 *

136          STKPUSH R7,R

000058 5030 C22E            00234   137+         ST    R3,STKSAV3

00005C 4830 C226            0022C   138+         LH    R3,STKCOUNT

000060 4930 C228            0022E   139+         CH    R3,STKSIZE

000064 47B0 C08C            00092   140+         BNL   Z0010

000068 5040 C232            00238   141+         ST    R4,STKSAV4

00006C 5020 C22A            00230   142+         ST    R2,STKSAV2

000070 8B30 0002            00002   143+         SLA   R3,2

000074 4120 C236            0023C   144+         LA    R2,THESTACK

000078 1847                         145+         LR    R4,R7

00007A 5043 2000            00000   146+         ST    R4,0(3,2)

00007E 4830 C226            0022C   147+         LH    R3,STKCOUNT

000082 4A30 C5A2            005A8   148+         AH    R3,=H'1'

000086 4030 C226            0022C   149+         STH   3,STKCOUNT

00008A 5840 C232            00238   150+         L     R4,STKSAV4

00008E 5820 C22A            00230   151+         L     R2,STKSAV2

000092 5830 C22E            00234   152+Z0010    L     R3,STKSAV3

153          STKPUSH HHW,H

000096 5030 C22E            00234   154+         ST    R3,STKSAV3

00009A 4830 C226            0022C   155+         LH    R3,STKCOUNT

00009E 4930 C228            0022E   156+         CH    R3,STKSIZE

0000A2 47B0 C0CC            000D2   157+         BNL   Z0011

0000A6 5040 C232            00238   158+         ST    R4,STKSAV4

0000AA 5020 C22A            00230   159+         ST    R2,STKSAV2

0000AE 8B30 0002            00002   160+         SLA   R3,2

0000B2 4120 C236            0023C   161+         LA    R2,THESTACK

0000B6 4840 C33A            00340   162+         LH    R4,HHW

0000BA 5043 2000            00000   163+         ST    R4,0(3,2)

0000BE 4830 C226            0022C   164+         LH    R3,STKCOUNT

0000C2 4A30 C5A2            005A8   165+         AH    R3,=H'1'

0000C6 4030 C226            0022C   166+         STH   3,STKCOUNT

0000CA 5840 C232            00238   167+         L     R4,STKSAV4

0000CE 5820 C22A            00230   168+         L     R2,STKSAV2

0000D2 5830 C22E            00234   169+Z0011    L     R3,STKSAV3

170          STKPUSH FFW

0000D6 5030 C22E            00234   171+         ST    R3,STKSAV3

0000DA 4830 C226            0022C   172+         LH    R3,STKCOUNT

0000DE 4930 C228            0022E   173+         CH    R3,STKSIZE

0000E2 47B0 C10C            00112   174+         BNL   Z0012

0000E6 5040 C232            00238   175+         ST    R4,STKSAV4

0000EA 5020 C22A            00230   176+         ST    R2,STKSAV2

0000EE 8B30 0002            00002   177+         SLA   R3,2

0000F2 4120 C236            0023C   178+         LA    R2,THESTACK

0000F6 5840 C336            0033C   179+         L     R4,FFW

0000FA 5043 2000            00000   180+         ST    R4,0(3,2)

0000FE 4830 C226            0022C   181+         LH    R3,STKCOUNT

000102 4A30 C5A2            005A8   182+         AH    R3,=H'1'

000106 4030 C226            0022C   183+         STH   3,STKCOUNT

00010A 5840 C232            00238   184+         L     R4,STKSAV4

00010E 5820 C22A            00230   185+         L     R2,STKSAV2

000112 5830 C22E            00234   186+Z0012    L     R3,STKSAV3

187          STKPUSH FFW,A

000116 5030 C22E            00234   188+         ST    R3,STKSAV3

00011A 4830 C226            0022C   189+         LH    R3,STKCOUNT

00011E 4930 C228            0022E   190+         CH    R3,STKSIZE

000122 47B0 C14C            00152   191+         BNL   Z0013

000126 5040 C232            00238   192+         ST    R4,STKSAV4

00012A 5020 C22A            00230   193+         ST    R2,STKSAV2

00012E 8B30 0002            00002   194+         SLA   R3,2

000132 4120 C236            0023C   195+         LA    R2,THESTACK

000136 4140 C336            0033C   196+         LA    R4,FFW

00013A 5043 2000            00000   197+         ST    R4,0(3,2)

00013E 4830 C226            0022C   198+         LH    R3,STKCOUNT

000142 4A30 C5A2            005A8   199+         AH    R3,=H'1'

000146 4030 C226            0022C   200+         STH   3,STKCOUNT

00014A 5840 C232            00238   201+         L     R4,STKSAV4

00014E 5820 C22A            00230   202+         L     R2,STKSAV2

000152 5830 C22E            00234   203+Z0013    L     R3,STKSAV3

204 *

205          STKPOP  R8,R

000156 5030 C22E            00234   206+         ST    R3,STKSAV3

00015A 4830 C226            0022C   207+         LH    R3,STKCOUNT

00015E 4930 C5A4            005AA   208+         CH    R3,=H'0'

000162 47D0 C186            0018C   209+         BNH   Z0014

000166 4B30 C5A2            005A8   210+         SH    R3,=H'1'

00016A 4030 C226            0022C   211+         STH   R3,STKCOUNT

00016E 8B30 0002            00002   212+         SLA   R3,2

000172 5020 C22A            00230   213+         ST    R2,STKSAV2

000176 5040 C232            00238   214+         ST    R4,STKSAV4

00017A 4120 C236            0023C   215+         LA    R2,THESTACK

00017E 5843 2000            00000   216+         L     R4,0(3,2)

000182 1884                         217+         LR R8,R4

000184 5840 C232            00238   218+         L   R4,STKSAV4

000188 5820 C22A            00230   219+         L   R2,STKSAV2

00018C 5830 C22E            00234   220+Z0014    L   R3,STKSAV3

221          STKPOP  FFW

000190 5030 C22E            00234   222+         ST    R3,STKSAV3

000194 4830 C226            0022C   223+         LH    R3,STKCOUNT

000198 4930 C5A4            005AA   224+         CH    R3,=H'0'

00019C 47D0 C1C2            001C8   225+         BNH   Z0015

0001A0 4B30 C5A2            005A8   226+         SH    R3,=H'1'

0001A4 4030 C226            0022C   227+         STH   R3,STKCOUNT

0001A8 8B30 0002            00002   228+         SLA   R3,2

0001AC 5020 C22A            00230   229+         ST    R2,STKSAV2

0001B0 5040 C232            00238   230+         ST    R4,STKSAV4

0001B4 4120 C236            0023C   231+         LA    R2,THESTACK

0001B8 5843 2000            00000   232+         L     R4,0(3,2)

0001BC 5040 C336            0033C   233+         ST     R4,FFW

0001C0 5840 C232            00238   234+         L   R4,STKSAV4

0001C4 5820 C22A            00230   235+         L   R2,STKSAV2

0001C8 5830 C22E            00234   236+Z0015    L   R3,STKSAV3

237          STKPOP  HHW,H

0001CC 5030 C22E            00234   238+         ST    R3,STKSAV3

0001D0 4830 C226            0022C   239+         LH    R3,STKCOUNT

0001D4 4930 C5A4            005AA   240+         CH    R3,=H'0'

0001D8 47D0 C1FE            00204   241+         BNH   Z0016

0001DC 4B30 C5A2            005A8   242+         SH    R3,=H'1'

0001E0 4030 C226            0022C   243+         STH   R3,STKCOUNT

0001E4 8B30 0002            00002   244+         SLA   R3,2

0001E8 5020 C22A            00230   245+         ST    R2,STKSAV2

0001EC 5040 C232            00238   246+         ST    R4,STKSAV4

0001F0 4120 C236            0023C   247+         LA    R2,THESTACK

0001F4 5843 2000            00000   248+         L     R4,0(3,2)

0001F8 4040 C33A            00340   249+         STH    R4,HHW

0001FC 5840 C232            00238   250+         L   R4,STKSAV4

000200 5820 C22A            00230   251+         L   R2,STKSAV2

000204 5830 C22E            00234   252+Z0016    L   R3,STKSAV3

253 *

Revised Code for the Macro STKINIT

Here is an expansion of the newer definition of STKINIT, 
which allows the stack size to be specified.

138          STKINIT 128

00004A 5030 C05E            00064   139+         ST R3,STKSAV3

00004E 1B33                         140+         SR R3,R3

000050 4030 C056            0005C   141+         STH R3,STKCOUNT

000054 5830 C05E            00064   142+         L  R3,STKSAV3

000058 47F0 C266            0026C   143+         B  L0009

00005C 0000                         144+STKCOUNT DC H'0'

00005E 0080                         145+STKSIZE  DC H'128'

000060 00000000                     146+STKSAV2  DC F'0'

000064 00000000                     147+STKSAV3  DC F'0'

000068 00000000                     148+STKSAV4  DC F'0'

00006C 0000000000000000             149+THESTACK DC 128F'0'

00026C 8B30 0000            00000   150+L0009    SLA R3,0