Sample Design:

A Controller for a Simple Traffic Light

Assumption: Two Linked Pairs of Traffic Lights

If one light is Green, the “cross light” must be Red.

Assumed Cycling Rules

One Light	Cross Light	Comments

Green	Red	Traffic moving on one street

Yellow	Red	Traffic on cross street must wait
			for this light to turn red.

Red	Red	Both lights are red for about
			one second.

Red	Green	Cross traffic now moves.

This is the basic sequence for a traffic light without turn signals or
		features such as an “advanced green”, etc.

Name the States

| State | Light 1 | Light 2 | Alias |
| --- | --- | --- | --- |
| 0 | Red | Red | RR |
| 1 | Red | Green | RG |
| 2 | Red | Yellow | RY |
| 3 | Red | Red | RR |
| 4 | Green | Red | GR |
| 5 | Yellow | Red | YR |

Step 1a: State Diagram for the System

Notation: L1L2, so RG  Light 1 is Red and Light 2 is Green

The six–state design is more easily implemented.

Step 1b: Define the State Table

| Present State |  | Next State |  |
| --- | --- | --- | --- |
| Number | Alias | Number | Alias |
| 0 | RR | 1 | RG |
| 1 | RG | 2 | RY |
| 2 | RY | 3 | RR |
| 3 | RR | 4 | GR |
| 4 | GR | 5 | YR |
| 5 | YR | 0 | RR |

At the moment, this is just a modulo–6 counter with unusual output.

We shall add some additional circuitry to allow for safety constraints.

The choice of Red – Red as state 0 is arbitrary, but convenient.

Step 2:  Count the States and 
Determine the Flip–Flop Count

There are six states, so we have N = 6.

Solve 2P–1 < N  2P for P, the number of flip–flops.

2P–1 < 6  2P gives P = 3, because 22 < 6  23.

We denote the states by Q2Q1Q0, because the symbol “Y” is taken to
		indicate the color Yellow.

Step 3: Assign a 3–bit Binary Number
to Each State

This is a modified counter, so the assignments are quite obvious.

| State | Q2 | Q1 | Q0 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 |
| 3 | 0 | 1 | 1 |
| 4 | 1 | 0 | 0 |
| 5 | 1 | 0 | 1 |

We have two possible additional states: 6 and 7.

Normally, these are ignored, but we consider them due to
		safety constraints.

Redefine the State Diagram to Add Safety

States 6 and 7 should never be entered.  Each is “RR” for safety.

Step 4a: Derive the Output Equations.

|  | Alias | Q2Q1Q0 | R1 | G1 | Y1 | R2 | G2 | Y2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | RR | 0 0 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | RG | 0 0 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 2 | RY | 0 1 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 3 | RR | 0 1 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 4 | GR | 1 0 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 5 | YR | 1 0 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 6 | RR | 1 1 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 7 | RR | 1 1 1 | 1 | 0 | 0 | 1 | 0 | 0 |

Here are the output equations

G1 = Q2Q1’Q0’	G2 = Q2’Q1’Q0
	Y1 = Q2Q1’Q0	Y2 = Q2’Q1Q0’
	R1 = (G1 + Y1)’	R2 = (G2 + Y2)’

Step 4a: Derive the Output Equations.
(page 2)

Here are the equations again.
		G1 = Q2Q1’Q0’		G2 = Q2’Q1’Q0
		Y1 = Q2Q1’Q0		Y2 = Q2’Q1Q0’
		R1 = (G1 + Y1)’		R2 = (G2 + Y2)’

We derive the Green and Yellow signals, which are easier.
We stipulate that if a light is not Green or Yellow, it must be Red.

Now add a safety constraint: If a light is Green or Yellow, the 
	cross light must be Red.

R1 = (G1 + Y1)’ + G2 + Y2, and 
	R2 = (G2 + Y2)’ + G1 + Y1

These equations may lead to a light showing two colors.
	This is obviously an error situation.

Step 4b: Derive the State Transition Table.

| Present State |  | Next State |
| --- | --- | --- |
|  | Q2Q1Q0 | Q2Q1Q0 |
| 0 | 0 0 0 | 0 0 1 |
| 1 | 0 0 1 | 0 1 0 |
| 2 | 0 1 0 | 0 1 1 |
| 3 | 0 1 1 | 1 0 0 |
| 4 | 1 0 0 | 1 0 1 |
| 5 | 1 0 1 | 0 0 0 |
| 6 | 1 1 0 | 0 0 0 |
| 7 | 1 1 1 | 0 0 0 |

Step 5: Separate the Table into Three Tables

| Q2 |  |  | Q1 |  |  | Q0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PS | NS |  | PS | NS |  | PS | NS |
| Q2Q1Q0 | Q2 |  | Q2Q1Q0 | Q1 |  | Q2Q1Q0 | Q0 |
| 0 0 0 | 0 |  | 0 0 0 | 0 |  | 0 0 0 | 1 |
| 0 0 1 | 0 |  | 0 0 1 | 1 |  | 0 0 1 | 0 |
| 0 1 0 | 0 |  | 0 1 0 | 1 |  | 0 1 0 | 1 |
| 0 1 1 | 1 |  | 0 1 1 | 0 |  | 0 1 1 | 0 |
| 1 0 0 | 1 |  | 1 0 0 | 0 |  | 1 0 0 | 1 |
| 1 0 1 | 0 |  | 1 0 1 | 0 |  | 1 0 1 | 0 |
| 1 1 0 | 0 |  | 1 1 0 | 0 |  | 1 1 0 | 0 |
| 1 1 1 | 0 |  | 1 1 1 | 0 |  | 1 1 1 | 0 |

Color added to emphasize the transitions of interest.

Step 6: Select the Flip–Flops to Use

Use JK flip–flops.  What a surprise!

The excitation table for a JK flip-flop is given again.

| Q(T) | Q(T + 1) |  | J | K |
| --- | --- | --- | --- | --- |
| 0 | 0 |  | 0 | d |
| 0 | 1 |  | 1 | d |
| 1 | 0 |  | d | 1 |
| 1 | 1 |  | d | 0 |

Step 7: Derive the Input Tables

| Flip-Flop 2 |  |  |  |  | Flip-Flop 1 |  |  |  |  | Flip-Flop 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PS | NS | Input |  |  | PS | NS | Input |  |  | PS | NS | Input |  |
| Q2Q1Q0 | Q2 | J2 | K2 |  | Q2Q1Q0 | Q1 | J1 | K1 |  | Q2Q1Q0 | Q0 | J0 | K0 |
| 0 0 0 | 0 | 0 | d |  | 0 0 0 | 0 | 0 | d |  | 0 0 0 | 1 | 1 | d |
| 0 0 1 | 0 | 0 | d |  | 0 0 1 | 1 | 1 | d |  | 0 0 1 | 0 | d | 1 |
| 0 1 0 | 0 | 0 | d |  | 0 1 0 | 1 | d | 0 |  | 0 1 0 | 1 | 1 | d |
| 0 1 1 | 1 | 1 | d |  | 0 1 1 | 0 | d | 1 |  | 0 1 1 | 0 | d | 1 |
| 1 0 0 | 1 | d | 0 |  | 1 0 0 | 0 | 0 | d |  | 1 0 0 | 1 | 1 | d |
| 1 0 1 | 0 | d | 1 |  | 1 0 1 | 0 | 0 | d |  | 1 0 1 | 0 | d | 1 |
| 1 1 0 | 0 | d | 1 |  | 1 1 0 | 0 | d | 1 |  | 1 1 0 | 0 | 0 | d |
| 1 1 1 | 0 | d | 1 |  | 1 1 1 | 0 | d | 1 |  | 1 1 1 | 0 | d | 1 |

Step 8: Derive the Input Equations

Here they are

J2 =  Q1  Q0	J1 =  Q2’  Q0	J0 = Q2’ + Q1’
	K2 = Q1 + Q0	K1 = Q2 + Q0	K0 = 1

There is no need to summarize the equations.

Step 10: Draw the Circuit