Modulo–4 Up–Down Counter

This is a counter with input.

If X = 0, the device counts up: 		0, 1, 2, 3, 0, 1, 2, 3, etc.
	If X = 1, the device counts down:	0, 3, 2, 1, 0, 3, 2, 1, etc.

Step 1a: Derive the state diagram and state table for the circuit.

Note two transitions between the state pairs: one is up and one is down.

Step 1b: Derive the State Table

| Present State | Next State |  |
| --- | --- | --- |
|  | X = 0 | X = 1 |
| 0 | 1 | 3 |
| 1 | 2 | 0 |
| 2 | 3 | 1 |
| 3 | 0 | 2 |

This is just a restatement of the state diagram.

Note: Two columns for the “Next State”.

Step 2: Count the States and
Determine the Flip–Flop Count

Count the States

There are four states for any modulo–4 counter.	N = 4

The states are simple: 0, 1, 2, and 3.

Calculate the Number of Flip–Flops Required

Let P be the number of flip–flops.

Solve 2P-1 < N  2P.		So 2P-1 < 4  2P and P = 2.

We need two flip–flops.

Step 3: Assign a unique P-bit binary number
(state vector) to each state.

Here P = 2, so we are assigning two–bit binary numbers.

Vector is denoted by the binary number Y1Y0.

| State | 2-bit Vector |
| --- | --- |
|  | Y1 Y0 |
| 0 | 0 0 |
| 1 | 0 1 |
| 2 | 1 0 |
| 3 | 1 1 |

Each state has a unique 2–bit number assigned.

Any other assignment would be absurd.

Step 4: Derive the state transition table
and the output table.

There is no computed output, hence no output table.

The state transition table uses the 2–bit state vectors

| Present State |  | Next State |  |
| --- | --- | --- | --- |
|  |  | X = 0 | X = 1 |
| 0 | 00 | 01 | 11 |
| 1 | 01 | 10 | 00 |
| 2 | 10 | 11 | 01 |
| 3 | 11 | 00 | 10 |

Step 5:Separate the state transition table into P tables,
one for each flip-flop.

Flip–Flop 1

| Flip-Flop 1 |  |  |
| --- | --- | --- |
| PS | Next State: Y1 |  |
| Y1Y0 | X = 0 | X = 1 |
| 0 0 | 0 | 1 |
| 0 1 | 1 | 0 |
| 1 0 | 1 | 0 |
| 1 1 | 0 | 1 |

Flip–Flop 0

| Flip-Flop 0: Y0 |  |  |
| --- | --- | --- |
| PS | Next State |  |
| Y1Y0 | X = 0 | X = 1 |
| 0 0 | 1 | 1 |
| 0 1 | 0 | 0 |
| 1 0 | 1 | 1 |
| 1 1 | 0 | 0 |

Step 6: Decide on the types of flip-flops to use.
When in doubt, use all JK’s.

Here is the excitation table for a JK flip–flop

| Q(T) | Q(T+1) | J | K |
| --- | --- | --- | --- |
| 0 | 0 | 0 | d |
| 0 | 1 | 1 | d |
| 1 | 0 | d | 1 |
| 1 | 1 | d | 0 |

Step 7: Derive the input table for each flip-flop

Flip–Flop 1

|  | X = 0 |  |  | X = 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Y1Y0 | Y1 | J1 | K1 | Y1 | J1 | K1 |
| 0 0 | 0 | 0 | d | 1 | 1 | d |
| 0 1 | 1 | 1 | d | 0 | 0 | d |
| 1 0 | 1 | d | 0 | 0 | d | 1 |
| 1 1 | 0 | d | 1 | 1 | d | 0 |

Flip–Flop 0

|  | X = 0 |  |  | X = 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Y1Y0 | Y0 | J0 | K0 | Y1 | J0 | K0 |
| 0 0 | 1 | 1 | d | 1 | 1 | d |
| 0 1 | 0 | d | 1 | 0 | d | 1 |
| 1 0 | 1 | 1 | d | 1 | 1 | d |
| 1 1 | 0 | d | 1 | 0 | d | 1 |

Question:  How do we produce equations for the J’s and K’s?

Step 8: Derive the input equations for each flip-flop

The equations are based on the present state and the input.

The input X produces a complication.

The simplest match procedure will lead to two equations
	for each flip–flop input: one for X = 0 and one for X = 1.

Use the “combine rule”

The rule for combining expressions derived separately
		for X = 0 and X = 1 is

X(expression for X= 0) + X(expression for X = 1).

Rationale:	Let F(X) = AX + BX
	When X = 0, F(X) = A and when X = 1, F(X) = B.

Input Equations for Flip–Flop 1

|  | X = 0 |  |  | X = 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Y1Y0 | Y1 | J1 | K1 | Y1 | J1 | K1 |
| 0 0 | 0 | 0 | d | 1 | 1 | d |
| 0 1 | 1 | 1 | d | 0 | 0 | d |
| 1 0 | 1 | d | 0 | 0 | d | 1 |
| 1 1 | 0 | d | 1 | 1 | d | 0 |

J1 = Y0	J1 = Y0’
	K1 = Y0	K1 = Y0’

Apply the “combine rule”

J1 = X’Y0 + XY0’ = X  Y0

K1 = X’Y0 + XY0’ = X  Y0

Input Equations for Flip–Flop 0

|  | X = 0 |  |  | X = 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Y1Y0 | Y0 | J0 | K0 | Y1 | J0 | K0 |
| 0 0 | 1 | 1 | d | 1 | 1 | d |
| 0 1 | 0 | d | 1 | 0 | d | 1 |
| 1 0 | 1 | 1 | d | 1 | 1 | d |
| 1 1 | 0 | d | 1 | 0 | d | 1 |

J0 = 1	J0 = 1

K0 = 1	K0 = 1

Apply the “Combine Rule”

J0 = X’1 + X1 = 1

K0 = X’1 + X1 = 1

Neither J0 nor K0 depend on X.  But Y0 does not depend on X.

Step 9:  Summarize the equations by
writing them in one place.

Here they are.

J1 = X  Y0		K1 = X  Y0
	J0 = 1				K0 = 1

Step 10: Draw the Circuit

As designed, it is:

Step 10: Draw the Circuit

Implemented with T Flip–Flops, it is:

One could also use a 4–register “one hot” design, with the input X
	used to determine the direction of the shift.